NORTHEASTERN REGIONAL HW INCINERATION CONTRACT SP4500-26-D-XXXX

- PERFORMANCE WORK STATEMENT (PWS) v.09/30/2025 (SWS)

1.0 REMOVAL PHASES

1.1 PRE-REMOVAL

1.1.1 CONTRACT LINE-ITEM NUMBER (CLIN) SELECTION CRITERIA

1.1.1.1 CLIN DISPUTES

1.1.2 PERMIT/SPECIAL REQUIREMENTS

1.1.2.1 REPORTING REQUIREMENTS

1.1.3 SHIPPING/DISPOSAL DOCUMENTATION

1.1.4 NOTIFICATIONS

1.1.5 CONVEYANCE VEHICLE OR TRAILER SECURITY

1.1.6 HOURS OF OPERATION

1.1.7 CONTRACT RELATED INFORMATION DISCLOSURE

1.1.8 DEFENSE BIOMETRIC IDENTIFICATION SYSTEM (DBIDS) INSTALLATION ACCESS

1.1.9 TREATMENT, STORAGE, AND DISPOSAL FACILITY (TSDF) CLOSURES AND OUTAGES

1.1.10 CONTRACTOR PERSONNEL

1.1.11 ADDING CONTAINERIZED WASTE TO ISSUED TASK ORDERS

1.2 REMOVAL

1.2.1 SPILL RESPONSIBILITY

1.2.2 SEGREGATION OF HAZARDOUS WASTE

1.2.3 CONTAINERS, SPECIAL PACKAGING, AND DEBRIS SIZE RESTRICTIONS

1.2.4 GOVERNMENT EQUIPMENT & PERSONNEL

1.2.5 TREATMENT OF HAZARDOUS WASTE ON GOVERNMENT FACILITIES

1.2.6 LOADING

1.2.7 WEIGHING OF ITEMS

1.2.7.1 TASK ORDER (TO) ADJUSTMENTS

1.2.8 DEFINITION OF CONTAINERIZED WASTE

1.3 POST REMOVAL

1.3.1 RETURN OF SHIPPING DOCUMENTATION

2.0 DISPOSAL

2.1 LITHIUM BATTERIES

2.2 POLYCHLORINATED BIPHENYLS (PCB) RELATED WASTES

2.3 MERCURY-CONTAINING WASTES

2.4 ENERGY RECOVERY

2.4.1 COMMODITY SPECIFIC RECOVERY REQUIREMENTS

2.5 DEMILITARIZATION

2.5.1 SUBMARINE TILES

2.5.2 OTHER MILITARY ITEMS

3.0 SPECIAL SERVICES

3.1 PERFORM OVERPACKING OR LOOSE PACKING SERVICES

3.2 PERFORM DDR LITHIUM BATTERY PACKAGING SERVICES

3.3 PROVIDE CERTIFICATE OF DISPOSAL OR DESTRUCTION (CD)

3.4 PROVIDE HAZARDOUS WASTE PROFILE SHEET (HWPS)

4.0 ACCEPTABLE PERFORMANCE LEVEL (APL)

4.1 APL TABLE

5.0 QUALIFIED FACILITIES AND TRANSPORTERS LISTS (QFL/QTL)

5.1 ADDING/UPDATING QUALIFIED FACILITIES AND TRANSPORTERS

5.2 FACILITY REMOVAL

5.3 TRANSPORTER REMOVAL

6.0 ADDITIONAL REQUIREMENTS

6.1 INDEMNIFICATION

6.2 PACKAGING, MARKING, LABELING AND PLACARDING

6.3 PLACES OF GOVERNMENT INSPECTION

6.4 CONTRACTOR REPRESENTATIVE

6.5 INCIDENTAL FEES

7.0 STANDARD PERFORMANCE TIME FOR WASTE REMOVALS AND PROVISION OF OTHER SERVICES

8.0 DOCUMENTATION REQUIREMENTS, DLA DISPOSITION SERVICES ACCEPTANCE, INVOICING, AND TRACKING

8.1 SUBMISSION OF DOCUMENTATION TO FACILITATE ACCEPTANCE FOR PAYMENT

8.2 ACCEPTANCE AND INVOICING

9.0 CONTRACT MINIMUM/MAXIMUM

10.0 CONTRACTOR ACCESS TO DLA DISPOSITION SERVICES SITES

10.1 CONTRACTOR PERSONNEL SECURITY REQUIREMENTS (H.14)

10.2 COMMON ACCESS CARD (CAC) PROCEDURES

10.2.1 DLA FORM 1728

10.2.2 DOD FORM 1172-2

10.2.3 CONTRACTOR RESPONSIBILITIES

10.3 CONTRACTOR CAC REPORTING REQUIREMENTS

10.4 CAC SECURITY COMPLIANCE

11.0 PERIOD OF PERFORMANCE

0.0 PERFORMANCE WORK STATEMENT (PWS)

a. This contract is referred to as the Northeastern Regional Hazardous Waste (HW) Incineration Contract.  Work will consist of the removal, transportation, incineration or other thermal treatment, and final disposal of Resource Conservation and Recovery Act (RCRA) regulated HW, non-RCRA wastes, Toxic Substance and Control Act (TSCA) regulated wastes, state regulated, and non-state regulated wastes generated by U.S. Department of War (DoW) and U.S. Coast Guard installations located in the 19 contiguous states of CT, DE, IL, IN, KY, MA, MD, ME, MI, NH, NJ, NY, OH, PA, RI, VA, VT, WI, WV, and Washington, D.C. A non-exhaustive list of potential pickup locations for waste removals and other limited services is provided as an attachment to this contract.  The awarded Contractor will conduct business solely with and on behalf of the U.S. Government.  The Government may add new pickup locations situated within the region at any time during the life of the contract.  The Contractor is advised that some requirements for information may be required before start of work under this contract per Sections 1.1.2, 1.1.8, and 5.1.

b. The period of performance for this contract consists of one (1) 30-month Base Period and one (1) 30-month Option Period.

c. This contract provides only for the removal of containerized wastes as defined in Section 1.2.8.  Pricing for containerized wastes is mostly based on container sizes.  Pricing for other containers is weight-based.  The standard period to perform the removal of containerized wastes and provision of other services under this contract is 30 business days.  A business day is defined as any day on which normal business is conducted.  It does not include weekends, federal holidays, or other days in which the Contractor is denied installation access due to force protection or other conditions.  Refer to Section 7.0 for additional information.

d. Bulk wastes requiring removal via rolloff boxes, vacuum boxes, sludge or dewatering boxes, dump trucks, tanker trucks, vacuum trucks, pump trucks, and trucks with vacuum-loaded or pump-loaded tanker trailers will not be turned in for removal under this contract.

e. Compressed gas cylinders, cartridges, vessels, and canisters will not be turned in for removal under this contract.

f. Services including equipment rental, provision of supplies, industrial cleaning, sampling, analysis, lab packing, bulking, consolidating, expedited performance, and expedited removals will not be ordered under this contract.  Overpacking, loose packing, and special packaging services for regulatory compliance and industry standard acceptance criteria are offered and may be ordered by the Government under this contract.  See Section 3.0 for more information.

g. This contract is a firm-fixed price, indefinite delivery/indefinite quantity (IDIQ) contract written as a performance-based contract and allows the Contracting Officer (KO) to make monetary deductions and/or a negative mark on the Contractor’s past performance record when an Acceptable Performance Level (APL) is not met.

h. Every Task Order (TO) issued under this contract will require the Contractor to provide waste removal within 30 business days at one (1) pickup location identified in or subsequently added to the awarded contract.  See non-exhaustive Pickup Locations List attachment for the current list of eligible pickup locations at the time of contract solicitation.

i. Minimum TO charges under this contract will be $900.00.  Only disposal CLINs (Contract Line-Item Numbers) will apply toward the minimum TO charge on every TO.  A surcharge for “Less Than Minimum Task Order” will be ordered by the Government on every TO not meeting or exceeding this amount.  CLIN 1093SV will have a firm-fixed price as shown in the Price Schedule of this contract.

j. The Government shall ensure that all items placed on a TO under this contract are packaged, marked, and labeled in accordance with (IAW) all applicable local, state, and federal laws and regulations relating to the generation, storage, and transportation of waste.

k. The Contractor shall provide all services necessary for the transportation, final treatment and disposal of the items listed in the price schedule IAW all local, state, and federal laws and regulations, and the terms and conditions of this contract.  These services shall include all necessary personnel, including subcontractors, labor, transportation, equipment, and the preparation, compilation, and submission of all documentation required by this contract.  All items, regardless of the condition, are being discarded by the Government and will require treatment and disposal of the waste identified by the ordered CLIN descriptions.

l. The six-character disposal CLIN consists of two parts.  The term “Base CLIN” refers to the first four characters of the CLIN.  The Base CLIN represents the waste category or series according to the waste profile for a particular waste stream.  The term “CLIN Suffix” refers to the last two characters of the CLIN.  The CLIN Suffix designates the unit of measure for purposes of tracking and Contractor payment.  For example, Base CLIN “FL16” represents “Non-Reactive paints and other coatings listing isocyanates, amines, or other polymerizable components in the ingredients” and CLIN Suffix “C4” designates the unit of measure as container up to 55-gallons in size.  As such, one (1) 55-gallon drum of this waste would be represented by ordering one (1) each of CLIN FL16C4.

m. Some CLINs ordered under this contract require the Contractor to provide a Certificate of Destruction or Disposal (CD).  The Contractor is advised to closely read all CLIN descriptions where such requirements are specified.  Unless explicitly written elsewhere in this PWS, the Contractor must only provide a CD when so reflected in the Price Schedule. See 3.3.

n. Publications and attachments applicable to this contract will be made available upon request.  During the solicitation and life of this contract, the Government may make requirement changes, additions, and/or deletions to those initially identified.

o. Generators are responsible for ensuring their containerized wastes sent for disposal via this contract are in proper Department of Transportation (DOT) shipping containers.

p. Required performance times for Contractor performance of services including waste removals are expressed in business days.  For timekeeping purposes in the performance of services, the day in which the TO is issued to the Contractor will constitute “Day 0” (zero), and the following business day will constitute “Day 1”.  All business days thereafter, whether in whole or in part, will constitute one (1) day for timekeeping purposes.  Refer to Section 7.0, Standard Performance Time for Waste Removals and Provision of Other Services.

q. For purposes of this contract, references to Department of Defense (DoD) and Department of War (DoW) are interchangeable and have the same meaning.

1.0 REMOVAL PHASES

The requirements for removal of wastes under this contract are divided into three main phases: Pre-Removal (1.1), Removal (1.2), and Post-Removal (1.3).  The Contractor and their Subcontractors must always perform in a prudent, conscientious, safe, and professional manner.  The Contractor shall ensure that its agents, employees, and Subcontractors involved in handling and packaging the hazardous waste be trained for the level of expertise required for the proper performance of the task and in the areas of chemical incompatibility, general first aid and spill response procedures.  The Contractor is required to provide only non-powered Material Handling Equipment (MHE) to move containers but has discretion to provide powered MHE if deemed advantageous to the Contractor.  Refer to Section 1.2.6 for additional information.  The Contractor shall provide appropriate Personal Protective Equipment (PPE) to Contractor and/or Subcontractor personnel to safely handle, move, and load wastes.  The Contractor agrees that all personnel and equipment used in the performance of this contract are subject to safety inspections by Government personnel while on federal property.

1.1 PRE-REMOVAL

1.1.1 CONTRACT LINE-ITEM NUMBER (CLIN) SELECTION CRITERIA

CLINs are assigned to waste streams by the Government and are designed around Environmental Protection Agency (EPA) RCRA regulations, treatment, and disposal technology standards.  DOT regulations, including Proper Shipping Name (PSN) and Hazard Class are not a basis for CLIN selection.  The waste description, generation process, physical state, and EPA Waste Code(s) are the primary considerations for CLIN selection.  CLIN selection begins with the identification of a Base CLIN category and sequence number.  It is selected, if available, from the Price Schedule, and is completed with the assignment of a CLIN Suffix.

a. Base CLIN categories and their descriptions used for waste disposal CLINs under this contract are as follows:

(1) Corrosives (CS) – Used for all corrosive wastes meeting the EPA definition of corrosive and carrying the EPA Waste Code D002, unless it is a Lab Pack/Loose Pack, or unless the waste is concurrently assigned a D001 (ignitable) or D003 (reactive) Waste Code.  Some corrosive acid oxidizers may carry the D001 Waste Code and be assigned a CS CLIN.

(2) Flammable/Combustible/Ignitable (FL) – Used for all ignitable and other than reactive or corrosive oxidizers carrying the D001 Waste Code, unless it is a Lab Pack or Loose Pack containing other than paint-related materials, or unless the waste is concurrently assigned a D003 (reactive) Waste Code.  FL CLINs may also be assigned to waste streams not carrying the D001 Waste Code but containing flammable/combustible/ignitable constituents such as organic solvents or chemicals.

(3) Lab Packs/Loose Packs (LP) – Used for most overpacked small containers of chemicals and consumer commodity-type products.  “Lab Pack” refers predominantly to the overpacking of multiple containers of like laboratory chemicals into a single outer container.  The chemicals must be of the same DOT hazard class and a detailed inventory must be affixed to the outer container.  Lab packing is a packaging method performed by a Contractor or Subcontractor under a separate service CLIN.  Although lab packing services are not offered under this contract, the removal and disposal of properly packaged lab packs is available to Generators under this contract.  “Loose Pack” refers predominantly to the overpacking of multiple containers of like consumer commodity products (e.g., paints, lubricants, adhesives, expired products, etc.) into a single outer container and does not require the preparation of a detailed inventory.  Generators of this waste may be authorized to loose-pack containers for turn-in.  Lab-packed and loose-packed drums and boxes containing smaller containers of the same type of wastes provide the best alternative when consolidation is impractical.  For purposes of disposal, Lab Packs and Loose Packs share the same Base CLIN depending upon the contents of the inner containers and their assigned EPA Waste Code(s) when applicable.

(4) Non-RCRA, Non-State and State-Regulated (NR) – Used for wastes that are not deemed hazardous under the United States EPA (USEPA) RCRA regulations and carry no USEPA Waste Code(s).  Some such wastes may be State regulated and may carry state-specific Waste Code(s).  The Contractor is required to comply with state regulations, as applicable, in the management of such non-RCRA hazardous wastes.

(5) PCB-Related (PC) – Used for all wastes containing or contaminated by a source of polychlorinated biphenyls (PCBs) that is equal to or greater than 50 parts per million (ppm) and regulated under the TSCA.  Some Generators of PCB-related wastes may choose to utilize “PC” Base CLINs, even when PCB concentrations fall below regulatory thresholds.  Additionally, some states regulate PCB-related wastes to a concentration threshold that is lower than 50 ppm.  The Contractor is required to comply with state regulations, as applicable, in the management of such PCB-related wastes.

(6) PFAS-Related (PF) – Used for various wastes containing or contaminated by a source of per- and polyfluoroalkyl substances (PFAS) including some Aqueous Film-Forming Foam (AFFF) wastes that are regulated under TSCA.  RCRA-regulated wastes containing PFAS are managed under “PF” CLINs according to their assigned EPA Waste Code(s).  At the time of the award of this contract, only PFAS-related wastes that are concurrently regulated under RCRA and require thermal destruction to meet Universal Treatment Standards per 40 Code of Federal Regulation (CFR) § 268.48 will be turned in for removal, treatment, and disposal.  During the life of this contract, non-RCRA regulated PFAS waste may be authorized for turn-in.

(7) Reactive (RX) – Used for various reactive wastes bearing the D003 Waste Code or potentially bearing reactive characteristics.  All wastes bearing the D003 Waste Code will be turned in for disposal using an “RX” or “LP” Base CLIN, except for waste also contaminated by a source of PCBs that is equal to or greater than 50 ppm.

(8) Toxic (TX) – Used for various toxic wastes that are not ignitable, corrosive, or reactive waste bearing the D001, D002, or D003 Waste Codes, apart from hexavalent chromium related waste bearing the D001 Waste Code exclusively due to Cr+6 oxidizing properties.

b. CLIN selection criteria are further based upon information found in the waste profile sheet to include the waste description, physical state of the waste, and the assignment of EPA Waste Code(s) to RCRA-regulated HW.  Every Base CLIN offered to Generators for use in the Price Schedule has corresponding permissible EPA Waste Code(s), unless the CLIN is for non-RCRA regulated waste.  Permissible Waste Codes for purposes of this contract are those EPA Waste Code(s) identified in the CLIN descriptions on the Price Schedule of this contract.  Permissible Waste Codes does not imply that any or all those waste codes will apply, but that no other waste code(s) may apply to waste streams assigned that CLIN.  CLINs showing D009* with the asterisk (*) as a permissible waste code must meet restrictions shown in Section 2.3 of this PWS.  Furthermore, the term “Listed" for purposes of this contract does not imply that all EPA Listed Waste Codes may apply.  Uncommon EPA Listed Waste Codes may be handled on a case-by-case basis, with consideration given to those Waste Codes that are permitted and accepted by the Contractor’s Treatment, Storage and Disposal Facility (TSDF) of choice.  The Government will ensure that all wastes being turned in for removal will carry only EPA Waste Code(s), if any, which are listed as permissible in the Price Schedule.  The Government is responsible for assigning proper profile-based CLINs to every waste stream being turned-in for removal.

A Base CLIN is generally authorized for use by the Government when the subject waste meets the waste description, physical state, and permissible EPA Waste Codes, as specified for that CLIN in the contract.  Base CLINs will be selected using the following Waste Codes and other descriptions as the order of precedence:

(1) PFAS- and/or PCB-Related (regulation varies by state)

(2) Reactive (D003)

(3) Ignitable/Oxidizer (D001), other than Cr+6 related waste bearing the D001 Waste Code exclusively due to the oxidizing properties of Cr+6.

(4) Corrosive (D002)

(5) Toxic (D004-D043)

(6) Acutely Hazardous (P-Listed)

(7) Leachate (F039)

(8) Electroplating-Related (F006-F012, F019)

(9) Spent Solvent (F001-F005)

(10) Other F-Listed

(11) U-Listed

(12) K-Listed

(13) State-Regulated Waste, Non-RCRA

(14) Non-RCRA, Non-State Regulated Waste

c. Base CLIN sequence number – Refer to the Master Disposal CLIN Catalog.

d. CLIN Suffix – Refer to the following CLIN Suffix Table:

| CLIN Suffix Table |  |  |  |  |
| --- | --- | --- | --- | --- |
| Containerized Wastes |  |  |  |  |
| CLIN Suffix→ | C1 | C2 | C3 | C4 |
| Container Size: Type: | Up to 5 Gal (DM/DF) | Up to 15 Gal (DM/DF) | Up to 30 Gal (DM/DF) | Up to 55 Gal (DM/DF) |
| U/M: | EA | EA | EA | EA |
| CLIN Suffix→ | C5 | C6 | C7 | C8 |
| Container Size: Type: | Up to 110 Gal (DM/DF) | Pallet, CYB, Tri-Wall Box | Containerized Pounds | Containerized Kilograms |
| U/M: | EA | EA | LB | KG |
| Special Laboratory Container Sizes |  |  |  |  |
| CLIN Suffix→ | ME | M1 | M2 | M3 |
| Container Size: Type: |  | Up to 1.25 Gal (DM/DF) | Up to 2.5 Gal (DM/DF) |  |
| U/M: |  | EA | EA |  |
| Pumpable Liquid Wastes Removed in IBC Totes |  |  |  |  |
| CLIN Suffix→ | T1 | T2 | T3 |  |
| Container Size: Type: | Up to 220 Gal (TP) | Up to 275 Gal (TP) | Up to 330 Gal (TP) |  |
| U/M: | EA | EA | EA |  |

e. NOTE TO GENERATORS: While some Generators have the option to manage some waste streams as Universal Waste (UW), Generators under this contract may be required to order disposal CLINs that provide for the proper treatment and disposal of waste based on the waste characteristics if it were otherwise managed as a HW.

(1) Example: UW aerosol cans that would otherwise carry corrosive (D002) or reactive (D003) EPA Waste Codes will be ordered under CLINs CS15C7 and RX01C7, respectively, and NOT under CLIN FL01C7, even when managed and turned in as UW with no EPA waste codes under state and/or federal regulations.  This is because corrosive and reactive wastes cannot typically be processed using fuels blending technologies for energy recovery as hazardous waste derived fuels. Generators will segregate their aerosol cans accordingly.  Other scenarios may apply.

f. Note on Fuels blending: Unless expressly prohibited by State or Federal regulations, fuels blending will meet thermal destruction requirements under this contract for wastes having a minimum energy value of 2,500 BTUs/Lb, including wastes consisting of or containing Petroleum, Oil, and Lubricants (POL), organic solvents, paints, and antifreeze.  See 2.4.1. for more on fuels blending.

1.1.1.1 CLIN DISPUTES

a. CLIN discrepancies noted on a TO either before or during a removal by the Contractor will be addressed immediately via the COR and the Generator.  If the COR and the Generator concur with the CLIN change request, a TO Modification will be issued.

b. The Government may unilaterally issue TO modifications for such reasons as quantity changes, noun description changes, CLIN changes, adding/deleting TO lines, extending TO end dates, etc.  The Contractor will be paid for the actual items and quantities removed per the awarded contract price schedule.  If upon receipt of a unilateral TO modification, the Contractor does not agree with the change or changes, the Contractor reserves the right to seek resolution with the KO.  These modifications are only against TOs.  No changes are made to the terms and conditions of the contract.

c. If the Contractor asserts after removal that an inappropriate CLIN has been assigned, the burden of proof lies with the Contractor.  The Contractor must provide documentation electronically to the KO and COR within 10 business days from the TSDF receipt date supporting the assertion through independently acquired laboratory analysis at the Contractor’s expense, TSDF fingerprinting results, Safety Data Sheet, and/or other written supporting documentation such as a TSDF “off-spec” notification.  Requests for a CLIN change without supporting documents will not be accepted.  After review of the documentation, the Contractor will be notified by the KO of the Government’s final determination.

d. The Contractor must ensure the waste in question is not treated or disposed before the Government has made a final CLIN determination.  If the subject waste has been treated or disposed of prior to the Government’s final determination, the Contractor will be deemed to have waived its objection to the CLIN assignment.

1.1.2 PERMIT/SPECIAL REQUIREMENTS

a. Without additional expense to the Government, the Contractor shall be responsible for paying all fees, preparing, or obtaining any necessary licenses, permits, notifications, waste profiles, or reports, which result from a Contractor’s transportation, treatment, or disposal decision.  This includes any fees associated with requirements of the EPA national electronic manifest (e-Manifest) system.  This includes acquiring any required permits or registrations necessary to operate on any installation listed in this contract and completing the Notification of Regulated Waste Activity form for the Generator(s) of hazardous waste to sign and file with the applicable state regulators for waste disposal.  This includes signing (on behalf of the Government) waste profiles requested by the TSDF.  All Contractor-signed waste profile sheets shall be written for the actual waste stream using the Generator’s profile for backup documentation.  Any Contractor-required waste profile certifications must be completed and signed by the Generator. All Generator forms or other means of notifications including correspondence with federal or state agencies on behalf of a Generator must be coordinated with and concurred with by the affected Generator prior to submission to any federal or state agency.  A copy of all forms and relevant documentation, including correspondence, shall be provided to the COR.

b. The Contractor may be required to attend special training, seminars, instructions, classes, safety orientations, etc., provided by the Government or to provide information to perform work or gain access to the installation.  Example: pass and ID requirements, antiterrorist training, Environmental Management System (EMS) policies, and/or forklift training.

c. The Contractor and any Subcontractor(s) shall use only the facilities and transporters currently (at the time) listed in the Defense Logistics Agency Disposition Services (DLADS) Qualified Facilities List (QFL) and Qualified Transporters List (QTL), per Section 5.0.

1.1.2.1 REPORTING REQUIREMENTS

a. If the Contractor uses a TSDF which requires the Generator(s) of hazardous waste to register with an out-of-state hazardous waste management facility prior to utilizing the facility, the Contractor must adhere to the requirements of Section 1.1.2.

b. The Contractor shall prepare and provide a Summary Manifest Report, including the completed manifest documents, to the Generator via the COR for filing with the State regulatory office(s).  This Summary Manifest Report will be provided in sufficient time to allow the Generators to file the reports within the time frames allotted by each state.  The required time frame will be established and documented when the Notification of Regulated Waste Activities form is submitted to the Generator via the COR, per Section 1.1.2.

c. The content of this Summary Manifest Report will be IAW the regulations of the state requiring the report.

1.1.3 SHIPPING/DISPOSAL DOCUMENTATION

a. All references to manifests in this contract refer to the appropriate shipping documents.   The Contractor shall use only a Non-Hazardous Waste Manifest or a Uniform Hazardous Waste Manifest for shipments of waste under this contract.  Bill of Lading (BOL) documents are prohibited for use under this contract.  Some states may require the use of state-specific waste codes and/or have unique requirements regarding the use of manifests.  The Contractor will comply with all such state requirements in the performance of this contract.  Prior to removal from a site identified as a pickup point in this contract, completed copies of all manifests and LDR notifications shall be furnished to the DLADS Field Activity coordinating the waste disposal.  All documentation shall be clearly marked with the name of the servicing DLADS Field Activity in addition to the Generator, contract, and TO number, as applicable.  Emergency response information and 24-hour emergency phone numbers shall be listed on the manifest.  If blocks for emergency information are not provided, this information shall be placed in the upper right-hand corner of each document.

b. Electronic Manifest System (e-Manifest system): Generators may continue to use paper manifests during their transition to fully electronic manifests in the e-Manifest system and the phase out of paper manifests.  If presented with a paper manifest, the Contractor will provide the Generator a paper copy signed by the initial transporter.  The Contractor shall provide the COR and the Generator with a copy of the completed form(s) for review by the appropriate Government official at least five (5) business days prior to removal.

1.1.4 NOTIFICATIONS

a. The Contractor shall notify the Contracting Officer Representative (COR) at least five (5) business days BEFORE attempting site visits, providing services, or removal efforts.

(1) In addition to the notification above for removal efforts, the Contractor shall provide the name of the driver, the driver’s Commercial Driver License (CDL) number and its expiration date, personnel badge expiration date, and truck badge expiration date (where applicable) to the COR or other Government representative at least 24 hours prior to removal.  The Contractor may opt to provide a list of approved, multiple drivers designated for pickups under this specific contract to the COR, or other Government representative prior to any attempted removals.  If this option is elected and the list is approved by the Government and maintained by the Contractor, the 24-hour notification requirement is waived for the remainder of the contract unless otherwise modified.

(2) If a driver, other than the one specified 24 hours in advance for the specific TO, or, one whose name does not appear on the optional list of approved, multiple drivers arrive for pickup, the COR or other designated Government representative will verify the personnel change with the Contractor before the pickup commences.  This may require a written notice or fax from the Contractor.

b. Defense Biometric Identification System (DBIDS) may be mandatory for access to some DOW facilities.  See Section 1.1.8 for further information.  Where DBIDS is utilized, the 24-hour advance notification is not required.  At some military installations, the Contractor and/or Subcontractors may be required to be escorted to and from the place of work.  This requirement is for both safety and physical security purposes.  Contractors, their employees, agents, and/or any Subcontractors are required to remain at the designated work area until released or escorted by the COR or other Government representative.

c. Notification requirements for an excusable delay request.  If at any time following the issuance of a written TO or anytime during the performance period, the Contractor determines standard removal times for any waste streams cannot be met due to unforeseen issues at the TSDF(s) beyond the Contractor’s control, the Contractor shall notify the KO and COR in writing via email by close of business (COB) the following business day, that a delay in removals under the subject TO will ensue.  Written notifications shall include the subject TO number, reason(s) for the delay, substantiable documentation from TSDF(s), and the estimated date/time of performance completion.  Substantiable documentation shall include written evidence showing earnest attempts were made by the Contractor to utilize at least two qualified alternative TSDFs to meet the performance requirements in the contract.  Failure by the Contractor to make a proper notification to the KO, as needed to render an informed decision regarding the Contractor’s asserted inability to perform timely due to factors beyond their control, shall constitute a waiver by the Contractor to exercise its excusable delay request.

(1) Following proper notification, if the KO determines the Contractor has met the burden of proof to demonstrate the delay is beyond the Contractor’s control, the following conditions apply:

(a) The Government reserves the right to issue a new (standalone) TO for impacted TO line(s) or modify and re-issue a previously issued TO.  Minimum TO charges will not apply to these TOs.

(b) The Contractor will provide biweekly TO updates to the KO and COR beginning 30 days after the new or modified TO is issued.  Updates shall include the status of the TO, the estimated time/date for removal, and further substantiable documentation that will provide the KO information to make an informed decision whether the delay remains beyond the control of the Contractor.  The requirement for TO updates will terminate when the Contractor’s requirements under the TO have been met.

(c) The Contractor will notify the KO and COR immediately upon determining that meeting standard removal times will resume.

(d) If a delay situation arises after issuance of a TO having a combination of impacted and non-impacted wastes, the KO, COR, and Contractor will mutually agree upon how best to address the situation with follow-on TO issuances and execute a contract modification memorializing said agreement.

1.1.5 CONVEYANCE VEHICLE OR TRAILER SECURITY

a. The Government will notify the Contractor when a conveyance vehicle or trailer must be locked and/or sealed due to Government policy or preference.  The Contractor will provide the lock and, if needed, the Government will provide the seal.  The Contractor will in all cases comply with state and federal laws, including DOT regulations as they pertain to conveyance vehicle and trailer security.

b. The Government has the right to request identification and/or endorsement(s) from the driver, even beyond what is identified in this contract.  The Government further has the right to refuse installation access and to stop Contractor/Subcontractor work of any kind, at any time, for any reason if/when the Government believes that a security risk may exist.  Any demurrage resulting from the driver verification process will not be grounds for Government reimbursement to the Contractor.

1.1.6 HOURS OF OPERATION

a. The service to be performed under this contract shall be performed during normal Government working days, as applicable for the site(s) on the TO, with specific office hours available from the COR on the TO, except for Federal holidays.  Work that cannot be completed by close-of-business for the TO site(s) shall be discontinued and resumed the next business day, unless approved by the KO.  Services will not be scheduled to be performed on a regular Federal holiday (see below), or one designated by Presidential proclamation.

| New Year’s Day | 1 January* |
| --- | --- |
| Martin Luther King's Birthday | 3rd Monday in January |
| Presidents Day | 3rd Monday in February |
| Memorial Day | Last Monday in May |
| Juneteenth | 19 June* |
| Independence Day | 4 July* |
| Labor Day | 1st Monday in September |
| Columbus Day | 2nd Monday in October |
| Veterans Day | 11 November* |
| Thanksgiving Day | 4th Thursday in November |
| Christmas Day | 25 December* |

*The holiday is observed on the preceding Friday or the following Monday when the holiday occurs on Saturday or Sunday, respectively.

b. Installation specific normal hours of operations for this contract are available upon request from the COR.  This contract is for various military and Government activities of which may have different work schedules.  Excluding weekends and observed Federal holidays, the Contractor agrees that, for those portions of the services provided on a government installation, the services will be provided during the normal hours of operation for the installation.

1.1.7 CONTRACT RELATED INFORMATION DISCLOSURE

The Contractor may not publicly release or share any data, reports, or other information relating to the contract with other firms or individuals who are not performing as subcontractors under the contract with a need to know and must keep such information confidential. Other than waste disposal data or other information that is required pursuant to laws and regulations implemented by regulatory authorities and applicable to all businesses, the release of any information pertaining to this contract is conditioned on the KO providing approval in writing prior to release.  The contractor shall immediately notify the KO upon receipt of any request for information from any entity, including regulatory authorities or the Press, to include any order to disclose information received from any competent judicial or administrative authority (e.g., federal, state, local or host nation officials or entities in a foreign country).  Disposition Services will respond promptly to advise the contractor regarding whether it authorizes the requested disclosure of information. The contractor is also prohibited from sharing information related to the contract in any forum or industry symposium without approval from the Contracting Officer.

1.1.8 DEFENSE BIOMETRIC IDENTIFICATION SYSTEM (DBIDS) INSTALLATION ACCESS

a. The DBIDS increases installation security and communications by receiving frequent database updates on changes to personnel and credential status, law enforcement warrants, lost/stolen cards, and force protection conditions.

b. DoW installations now require all Contractors and their representatives to be credentialed via enrollment in the DBIDS to gain access to these installations.  DBIDS replaces the former Rapid Gate access system and requires a security background check for all Contractors and their representatives.  The Contractor should plan accordingly.  For DBIDs enrollment, go to https://dbids-global.dmdc.mil/enroll.

c. Additional credentials and documentation may be required at some installations.  The Contractor will consult the COR for additional info regarding additional credentialing requirements.

d. During performance of the contract, the contractor shall be responsible for obtaining required identification for newly assigned personnel and for prompt return of credentials and vehicle passes for any employee who no longer requires access to the work site.

e. Upon completion or termination of the contract or expiration of the identification passes, the prime contractor shall ensure that all base identification passes issued to employees and subcontractor employees are returned to the issuing office.

f. Failure to comply with these requirements may result in withholding final payment.

1.1.9 TREATMENT, STORAGE, AND DISPOSAL FACILITY (TSDF) CLOSURES AND OUTAGES

Permanent or temporary closures or outages of the Contractor’s TSDF of choice, whether owned or subcontracted by the Contractor, does not alleviate the Contractor of its responsibility to perform on-time waste removals according to specified performance times and firm-fixed pricing in this contract.  Advanced notifications to the COR of scheduled TSDF outages for such reasons as maintenance standdowns may allow for flexibility in removal dates when agreed upon by the Government. Unless the COR or KO authorize in writing to accept delays of service due to a TSDF closure or outage, the Contractor shall utilize an alternative qualified TSDF when necessary to meet containerized waste removal requirements at no additional cost to the Government.  The Contractor is advised to always have an alternative qualified TSDF in place to ensure timely removals. Refer to Section 1.1.4 for excusable delay requests needed after the issuance of a written TO.

1.1.10 CONTRACTOR PERSONNEL

All Contractor personnel performing work under this PWS shall be qualified via training and experience, commensurate with the duties being fulfilled at least to the minimum requirements under relevant 29 CFR-OSHA, 40 CFR-EPA, 49 CFR-DOT, EM 385 standards, and updates thereto.  The Contractor shall be responsible for all initial and refresher training required of Contractor personnel and it shall be completed in the appropriate timeframes.  Upon request by the Government, the Contractor shall provide proof that these trainings were completed.

1.1.11 ADDING CONTAINERIZED WASTE TO ISSUED TASK ORDERS

The Government will be allowed to add limited containerized wastes to issued TOs up to five (5) business days prior to regularly scheduled removals.  The addition of such containers to an active TO will be limited to those waste streams having been fully profiled with the Contractor’s designated TSDF and having their associated CLINs already existing on the Price Schedule.  If the Contractor has already provided the COR and the Generator with a copy of the completed manifest per Section 1.1.3, the Contractor will edit the manifest on or before the time of removal.

1.2 REMOVAL

1.2.1 SPILL RESPONSIBILITY

a. The Contractor and/or Subcontractor shall have a serviceable, compatible, and operable spill kit(s) on-site to capture and contain a spill or release during waste removals.  The Contractor is solely responsible for cleanup and proper disposal of all spills or leaks occurring during the performance of this contract because of, or are attributed to, the actions of its agents, employees, or Subcontractors to the satisfaction of the Government and at no additional cost to the Government.  Hazardous waste spill cleanups that occur because of, or are attributed to, the actions of its agents, employees, or Subcontractors shall be removed off-site within 72-hours of the cleanup.  Under extenuating circumstances and with approval of the host’s Environmental Office, hazardous waste spill cleanups may be allowed to move to a permitted Satellite Accumulation Area until it can be shipped off-site.  The Contractor shall prepare a separate manifest for the spill cleanup removals.

b. The Contractor shall report all such spills or leaks, regardless of their quantity, to the COR (if not already present) and the KO immediately upon discovery.  The Contractor shall adhere to all Host or Installation spill reporting and clean-up requirements.  A written follow-up report shall be submitted to the KO not later than 24 hours after the initial telephonic report.  The written report shall be in narrative form and as a minimum include the following:

(1) Description of material spilled (including identity, quantity, manifest number, etc.).

(2) Whether quantity spilled is EPA/state reportable, and if so, whether it was reported.

(3) Exact time and location of spill, including a description of the area involved.

(4) Containment procedures initiated.

(5) Summary of any communications Contractor has with anyone other than the KO.

(6) Description of clean-up procedures employed or to be employed at the site including disposal location of spill residue.

1.2.2 SEGREGATION OF HAZARDOUS WASTE

All items collected on this contract must be segregated according to the current EPA Hazardous Waste Compatibility Chart and kept physically separate from any containers not associated with this contract until the initial TSDF is reached.  The items must be so marked such that they are readily linked to this contract throughout this period.  The Contractor may bring other waste onsite while conducting milk runs, or picking up waste at multiple locations, provided the waste is listed on the current TO(s) being picked up.

1.2.3 CONTAINERS, SPECIAL PACKAGING, AND DEBRIS SIZE RESTRICTIONS

a. The Contractor is responsible for special packaging and costs associated with unique container packaging requirements for acceptance at the Contractor’s preferred TSDF. For purposes of this contract, unique container packaging for TSDF acceptance shall refer to (1) packaging not specifically required under current DOT regulations, (2) not specifically required per the PWS or Price Schedule in this contract, and (3) not sufficiently demonstrated by the Contractor to the KO’s satisfaction to be a widely required packaging standard used by the TSDF industry.

b. Generators are responsible for ensuring their containers and packages (1) comply with current DOT regulations, (2) conform to Generator packaging requirements identified in this contract, and (3) conform to industry packaging standards when sufficiently demonstrated as such by the Contractor and confirmed by the KO.  Containers must be in good condition and compatible with their contents per 40 CFR Parts 265.171 and 265.172. Generators of ANY reactive wastes are highly encouraged to consult with the Contractor and COR regarding special Generator packaging requirements BEFORE packaging their reactive wastes for turn-in.  Improperly packaged wastes will be repackaged at the Generator’s expense.

c. Some waste streams turned in under this contract require special packaging by the Generator prior to turn-in for removal.  Some examples of Generator special packaging include:

(1) CLIN BA17 – Damaged, Defective, or Recalled (DDR) lithium batteries will be packaged IAW 49 CFR §173.185(f) or updates thereto. Generators may choose to have the Contractor package these items under special services CLIN 1221SV. See 3.2.

(2) CLINs RX08 and RX09 – Air/Water Reactive metals. Unless otherwise specified by the Contractor prior to removal, Generator special packaging requires the waste to be submersed under food grade mineral oil in a poly/plastic “inner” container up to 5 gallons in size.  The inner container will be overpacked in a metal “outer” container up to 55 gallons in size, with vermiculite between the two containers. The maximum gross weight of the combination package will not exceed 100 pounds. The CLIN suffix (C1-C4) will be based on the size of the outer container.

(3) Road flares and other incendiary items authorized for turn-in are packed under water, unless advised otherwise by the Contractor.

d. Generators will ensure that liquid wastes capable of expansion due to extreme heat or freezing are suitably containerized prior to turn-in.  Once a container has been removed from the Government installation, the Contractor is responsible for any mitigation efforts to prevent or contain leakages associated with bulging, damaged, or ruptured containers up to and including delivery to their designated TSDF.

f. When a “C6” CLIN suffix is ordered by the Government and the package contains (up to 18) 5-gallon pails, the Government will pay only for 1 EA pallet, CYB, or military tri-wall box, as applicable.  However, for manifesting purposes, the Government will inform the Contractor of the quantity of 5-gallon pails comprising the package, either on the TO or by other written communication.  See Section 1.2.8 for additional information on usage of the “C6” CLIN suffix.

g. Special packaging will be used only to provide safe storage and transport.  It will not be used to perform treatment or to remove any EPA Waste Codes.

i. The Contractor will, when possible, preemptively notify the COR and/or Generators directly when Generator special packaging will be required prior to Generators packaging for turn-in.  Generators are strongly encouraged to consult the COR (and hence the Contractor) regarding special packaging requirements prior to packaging potentially subject items for turn-in.  When warranted, the COR will consult the Contractor and advise the Generator.

j. If the Government questions whether a Contractor's special packaging requirement aligns with an industry standard practice, the COR will consult with the KO for guidance. The KO may require the Contractor to provide evidence that similar TSDFs impose the same acceptance criteria. DLA may also conduct market research to determine if the requirement is a widely used TSDF industry standard. DLA will not provide detailed written results of this market research.

k. The Government shall purchase and provide containers except when DDR lithium-ion packaging services are ordered under CLINs 1221SV-1224SV, where the Contractor must provide the containers.

l. Metal debris size restrictions apply to this contract.  Generators may turn-in properly packaged heavy gauge metal debris waste items, such as pipes, filters, equipment, and tools with the following restrictions, which may be waived by the Contractor on a case-by-case basis:

- Maximum length: 24 inches

- Maximum diameter/width: 12 inches

- Maximum thickness for pipes: 0.5 inches

- Maximum thickness for other metal items: 2 inches

- Maximum weight per metal piece: 50 pounds

- Maximum container size: 55-gallons (limit 10 per shipment)

1.2.4 GOVERNMENT EQUIPMENT & PERSONNEL

The Government shall not be required but is encouraged to furnish equipment and/or personnel to assist the Contractor with loading and weighing of items turned in under this contract.  Refer to Sections 1.2.6 and 1.2.7.

1.2.5 TREATMENT OF HAZARDOUS WASTE ON GOVERNMENT FACILITIES

a. Treatment of hazardous waste (including solidification) on Government facilities is not permitted.  Treatment is defined as any process which meets the definition of treatment under 40 CFR § 260.10 and/or the Facility Permit.

b. The Contractor shall not drain and/or flush PCB items at Government facilities.  Draining will be allowed only to prevent leaking and to meet DOT regulations.

1.2.6 LOADING

a. The Contractor is responsible for loading containers (including palletized containers and totes) onto conveyance trucks and trailers for removal from the installation. Due to the geographical scope of this contract, the Contractor may, but is not required to, furnish gas or electric-powered Material Handling Equipment (MHE) for moving or loading containers. The Contractor may use pallet jacks, hand carts, dollies, or other suitable equipment for these tasks. The Government is not responsible for blocking, chocking, bracing, lashing, banding, or otherwise securing cargo on Contractor-provided conveyances.

b. During normal operating hours, the Government may offer to assist the Contractor with loading at pickup locations if personnel and equipment are available. If the Contractor accepts the offer, the Government will make the initial placement of items onto the Contractor's conveyance, as determined by the Government (i.e., "tailgate" the items).

c. The Government is responsible for staging containers for removal at ground or loading dock level, as appropriate. Furthermore, the Government must ensure that waste scheduled for removal is readily accessible to the Contractor. "Accessible" in this context means unobstructed and does not require the relocation of other containers or objects not scheduled for removal under the subject TO.

1.2.7 WEIGHING OF ITEMS

a. Weight Verification: All items will be weighed before removal. The Contractor may accept weights provided by the Government. Otherwise, the Contractor will provide portable scales for weighing items, including containers, pallets, and dunnage. Scales must demonstrate reasonable weight accuracy to the Government representative before use.

b. Government Scales: If available, operable, and authorized by the Government, Government-owned scales may be used by mutual agreement. The Contractor is responsible for contacting the COR to determine the availability of Government scales.

c. Scale Calibration and Accuracy: All scales used for this contract must be calibrated according to applicable regulations. The Contractor is responsible for ensuring the operability and accuracy of non-Government scales through certification or demonstration. The Government is responsible for ensuring the operability and accuracy of Government scales through certification or demonstration. The use of commercial scales is prohibited under this contract.

1.2.7.1 TASK ORDER (TO) ADJUSTMENTS

a. The Government will be allowed to add items having pre-approved waste profiles to an open TO up to five (5) full business days prior to the scheduled removal date.  The Government may not add items to an open TO with fewer than five (5) full business days remaining before the scheduled removal unless otherwise agreed upon by the Contractor.  Waste streams not having approved profiles in place with the Contractor prior to removal may be rejected and subsequently require removal from the TO at or before the time of removal.

b. Initial TO weights and volumes are often estimated.  The Contractor should expect that TO weights will require adjustment upward or downward as removals take place and the actual weights being removed are determined.  Therefore, the Government has the unilateral right to increase or decrease the TO weight in those cases where the Government has underestimated or overestimated the weight ordered for removal, allowing the Contractor to complete the performance.  In cases of adjustments resulting in a decreased weight, these decreases are not subject to FAR 52.212-4, termination for convenience coverage.  In addition, it is agreed that it is acceptable for the Government to issue TO modifications unilaterally for quantity changes, noun description changes, CLIN changes, adding/deleting lines, extending TO end dates, etc.  The Contractor will be paid for the actual items and quantities removed per the awarded contract price schedule.  If upon receipt of a unilateral TO modification, the Contractor does not agree with the Government’s change(s), the Contractor reserves the right to seek resolution through the KO.  These modifications pertain only to issued TOs, and no changes will be made to the terms and conditions of the contract.

1.2.8 DEFINITION OF CONTAINERIZED WASTE

a. For purposes of this contract, the term containerized waste refers to waste that is typically transported by a box truck or semi-truck and trailer or van, including waste contained in drums, totes, cubic yard boxes, military tri-wall boxes, cubic yard bags/super sacks, or palletized waste.

b. Containers shall be counted, weighed, and documented against the TO by the Contractor prior to removal.  Although most containerized CLINs are priced by the container size (CLIN Suffixes C1-C6, and T1-T3), weights must still be documented for other purposes such as manifesting and Generator reporting.

c. Drums larger than 55 gallons ordered under CLIN Suffix “C5” are to be used only for overpacking smaller drums when the smaller drum(s) are leaking or have the potential to leak.  Generators shall be required to pay the Contractor based on the size of the overpack/salvage drum, and not on the size of the inner, overpacked drum(s).

d. “CYB” (CLIN Suffix “C6”) refers to Cubic Yard Boxes, Cubic Yard Bags or Super Sacks, Gaylord Boxes, and Military Tri-Wall Boxes.

e. “Pallet” (CLIN Suffix “C6”) refers to a flat wooden, plastic or metal transport structure not more than 48” L x 48” W that supports goods in a stable manner while being lifted by a forklift, pallet jack, front loader, or other lifting device.  Skids may be used in lieu of pallets when well suited to support the cargo and are accessible to movement via typical MHE. Containers/items placed on a pallet are often secured with dunnage, i.e., banding straps, stretch or shrink wrap for stability in movement.  The sum weight of the dunnage, pallet, and palletized waste cargo will collectively comprise the weight of the waste and shall not exceed a height of 42” or a gross weight of 1,600 pounds unless otherwise authorized by the Contractor.

f. The cost to the Generator for a CYB and Pallet are the same when the CLIN Suffix C6 is used and packaged IAW current DOT shipping requirements.  However, this does not imply that both packaging methods are always appropriate for wastes assigned a CLIN Suffix of C6.  Safety and DOT shipping regulations shall always take precedence.

g. Containers of the same Base CLIN and no larger than 5 gallons in size, may be palletized or placed in a CYB to assign the CLIN Suffix C6.  This provision allows for up to eighteen (18) 5-gallon pails per pallet or cubic yard box.  For manifesting purposes, the Government shall inform the Contractor of how many 5-gallon pails comprise the package.  No such container quantity limit shall apply to containers that are smaller than 5 gallons in size or to consumer commodities packaged in their original packaging.  Containers larger than 5 gallons in size will not be placed in a CYB for purposes of consolidating them into a single container.  Containers of all sizes may be placed on pallets and skids for ease of storage and movement, but containers larger than 5 gallons, other than consumer commodities in their original packaging which may be larger than 5 gallons in size, will have the quantity and CLIN assigned as appropriate to each container on the pallet.

h. Most containerized wastes are assigned a CLIN Suffix of C1-C6 and T1-T3.  These CLIN Suffixes correspond to a range of different container sizes and are ordered as each “EA”.  Some CLINs for containerized wastes are designated with CLIN Suffix C7 and are ordered in pounds “LB”.  CLINs for containerized PCBs are designated with CLIN Suffix C8 and are ordered in kilograms “KG”.

i. “Tote” (CLIN Suffixes T1, T2, and T3) refers to a portable tank or Intermediate Bulk Container (IBC) designed for storage and transport of bulk liquids.  For purposes of this contract, liquid wastes that are removed in totes are considered containerized wastes.  Tote sizes are commonly designed in increments of 55 gallons, i.e., 220, 275, and 330 gallons.  CLIN Suffixes T1, T2, and T3 shall apply to these tote sizes, respectively.  The unit of measure for these CLIN Suffixes will be Each (EA), regardless of the actual volume of liquid contained in the tote, and not to exceed the design capacity.  Totes shall not be returned to the Government and disposal pricing will include any associated processing fees, if applicable.

1.3 POST REMOVAL

1.3.1 RETURN OF SHIPPING DOCUMENTATION

a. At the time of removal, the Contractor shall provide the Generator with a complete and accurate, transporter-signed copy of each manifest that is used for the waste removal.  If a Government representative is not present at the time of removal, the Contractor shall provide the COR with a copy of the transporter-signed pickup manifest(s) within three (3) full business days following the removal.

b. Unless explicitly prohibited, the use of transfer facilities is authorized during transit of waste to the designated facility.  A transfer facility is any transportation related facility including loading docks, parking areas, storage areas, and other similar areas where shipments of hazardous waste are held during the normal course of transportation, per 40 CFR § 260.10.  The Contractor will ensure the 10-day holding time is not exceeded at any transfer facility.

c. The Contractor will provide return manifests, i.e., an electronic copy of the designated (receiving) facility-signed manifests to the appropriate CORs via email.  CORs will forward return manifests to Generators.

d. The Contractor will electronically submit an Invoicing packet to DLA Disposition Services IAW Section 8.0, which will include a copy of manifest(s) signed by the first TSDF, except for TOs that include the disposal of wastes requiring the Contractor’s provision of a CD, which require an audit trail to the final TSDF.

e. The Contractor will submit/upload completed HW manifests to the EPA e-Manifest system.  If a TSDF-signed copy of the manifest will not be submitted within 45 calendar days following the date of waste removal, the Contractor will notify the COR and Generator of the status of the waste prior to Day 45.  If a TSDF-signed copy of the manifest won’t be submitted within 60 days following the date of removal, the Contractor will provide impacted Large Quantity Generators (LQG) with an Exception Report prepared to readily submit in the e-Manifest system prior to Day 60.

f. If the Contractor is unable to deliver the waste to the designated or alternate facility identified on the manifest(s) per 40 CFR § 263.21(a), the COR and Generator will be contacted for disposition and manifest revision instructions per 40 CFR § 263.21(b).  The Contractor shall make reasonable attempts to deliver the waste to a qualified alternate facility, rather than returning the waste to the Generator. In the event of rejected shipments, or where the shipment is undeliverable, the Contractor shall provide the Generator with a written narrative of the incident, including details of all attempts to provide delivery to a qualified TSDF.

2.0 DISPOSAL

a. The Contractor's responsibility for disposal services is fulfilled upon delivery of manifested waste to a qualified TSDF, receiving a signed manifest with no subsequent rejection, and returning shipping documents per Section 1.3.1.  Further shipment documentation is not required unless provision of a CD is specified in the disposal CLIN description or when CLIN 1300SV was ordered per Section 3.3.

b. While TSDFs are required under their Permit(s) to prevent the receipt of unauthorized wastes, the Contractor is responsible to ensure that properly profiled wastes are shipped only to those facilities that hold the appropriate Permit(s) to receive such wastes.

c. Regardless of the ordered disposal CLIN and its associated description, the Contractor shall comply with all State and Federal regulations.  Some wastes may be state regulated, where such regulations are not cited or regarded in the CLIN descriptions.  It is the responsibility of the Contractor to notify the Government if an ordered CLIN requires a treatment or disposal technology that conflicts with any such regulation(s).

2.1 LITHIUM BATTERIES

a. Lithium and lithium-ion batteries turned in under this contract will vary in design and chemistry.  Lithium primary batteries are generally single-use, non-rechargeable batteries.  Lithium-ion batteries are classified as secondary and are generally rechargeable.  For purposes of this contract, both lithium primary and lithium-ion secondary batteries are collectively referred to as “lithium batteries”.

b. Only Damaged, Defective, or Recalled (DDR) lithium batteries requiring RCRA incineration will be turned in for removal under this contract, using disposal CLIN BA17C7. The Unit of Measure is pounds (LB) and shall reflect the gross weight of the entire package. These batteries or battery residues require Generator special packaging IAW 49 CFR 173.185(f) or updates thereto. Generators may order this packaging service under CLIN 1221SV. See 3.2.

c. When multiple lithium batteries have been melted or fused together due to heat of reaction or are affixed to a board or piece of equipment and cannot be safely removed for individual packaging, the battery “mass” will be packaged as one battery with at least the same packaging requirements as a single DDR battery.

d. Even when DDR lithium batteries are thought to be fully discharged or rendered inert due to reaction, the D003 Waste Code will be assigned the waste. DDR lithium batteries can be managed as Universal Waste (UW) only when the battery casing hasn’t been breached.

2.2 POLYCHLORINATED BIPHENYLS (PCB) RELATED WASTES

a. PCB-related wastes under this contract shall be transported and disposed only within the Continental United States (CONUS).

b. The Contractor shall manage all items in compliance with 40 CFR § 761.218 and 49 CFR § 761.60 and all referenced parts.  When conducted IAW 40 CFR Part 761, the Contractor may drain and flush equipment and other items of free liquids containing PCBs for recycling or disposal of the drained equipment or items at a Toxic Substances Control Act (TSCA) permitted landfill.  The drained PCB liquid will be thermally destroyed at a TSCA permitted incinerator.  When and where authorized under State regulation, dechlorination treatment of PCB fluids is authorized under this contract prior to thermal destruction of the treated fluid.

c. CDs for all PCB-related wastes containing or contaminated by a source that is equal to or greater than 50 ppm PCBs (or 5 ppm in California and 2 ppm in Washington State), shall be provided to the Generator and COR within 30 calendar days from the date of destruction.  Where states regulate PCBs at a lower concentration threshold, the Contractor shall comply accordingly.  The Contractor will not be authorized payment until required CD(s) are received by the COR and Generator ordering the certification.

2.3 MERCURY-CONTAINING WASTES

a. Wastes containing mercury and assigned the EPA Waste Code D009 are required to meet restrictions specified herein. These restrictions are imposed to prevent the prohibited use of dilution as a substitute for treatment, per 40 CFR § 268.3, as it relates to the combustion of RCRA-regulated Inorganic Metal Bearing Hazardous Wastes (IMBHW).

b. For purposes of this contract, no waste stream assigned EPA Waste Code D009 will be turned in for removal unless the Generator certifies to the satisfaction of the Contractor that the subject waste stream meets at least one of the following exceptions at the point of generation:

- The waste contains hazardous organic constituents or cyanides at levels exceeding the constituent-specific treatment standard found in 40 CFR § 268.48.

- The waste consists of organic, debris-like materials (e.g., wood, paper, plastic, or cloth) contaminated with IMBHW.

- The waste, at point of generation, has a heating value of at least 5,000 BTUs/Lb.

- The waste is co-generated with wastes for which combustion is a required method of treatment.

- The waste is subject to Federal and/or State requirements necessitating reduction of organics (including biological agents); or

- The waste contains greater than 1% Total Organic Carbon (TOC).

Additionally,

- The waste package must contain less than 400 ppm (0.04%) mercury (Hg), calculated based on the gross weight of the container; and

- The waste must not originate from or be contaminated by a PCB source containing equal to or greater than 50 ppm PCBs.

2.4 ENERGY RECOVERY

2.4.1 COMMODITY SPECIFIC RECOVERY REQUIREMENTS

Fuels blendable wastes will contain a minimum heating value of 2,500 BTUs per pound of waste.  For lab pack and loose pack containers having multiple small containers packed within, the BTU value is based on the overall contents, and not each individual small container.  Since BTU values are not always known, the waste profile information may be used to determine eligibility for fuels blending.  Latex paint which has not been diluted with water will be considered fuels blendable.  Oil-based paint and its paint-related materials (PPRM), as well as other organic-based, flammable, combustible, and ignitable wastes, are also commonly fuels blendable.  In general, for purposes of this contract, PPRM will be defined as any mixture of pigment and a suitable carrier liquid that forms a closely adherent coating when spread on a surface, as well as their respective thinners and dried solids.  This may include but is not limited to paints and paint thinners, stains, varnishes, shellacs, and other coatings.  It does not include discarded items related to painting activities, e.g. paint brushes, rollers, pans, tape, PPE, etc., sometimes referred to as paint and paint-related waste (PPRW).

a. Base CLINs FL05-FL10 and LP14 were designed around fuels blending; a method used to blend and utilize combustible hazardous wastes in such a way as to meet energy (BTU) and chlorine specifications at RCRA Permitted incinerators and cement kilns as an alternative fuel source.  Wastes turned in under these CLINs will be thermally destroyed and presumed by the Government to be fuels blended for energy recovery, whether ultimately processed as an energy source at a RCRA Permitted incineration or cement kiln facility.

2.5 DEMILITARIZATION

a. When waste items requiring Demilitarization or “DEMIL” are ordered by the Government under this contract, the items shall be thermally destroyed.

b. For DEMIL-required items, the Contractor is required to dispose of these items per the CLIN description and provide all documentation, in addition to the documentation requirements of Section 8.0 for invoicing.  The Contractor shall ensure appropriate DEMIL/destruction occurs within 90 calendar days of pickup and provide a CD within 30 calendar days following destruction.  Documentation shall be returned to the Generator, as listed in Block 5 of the manifest, and the COR.

2.5.1 SUBMARINE TILES – CLIN TX99

a. Submarine tiles, including those designated as Special Hull Treatment (SHT) tiles are unique military items that require DEMIL at RCRA-permitted incineration facilities. The Contractor shall transport the property directly from the pickup location to the incineration facility. When interim transfer or storage is required, the Contractor shall utilize only their own company’s network of secure facilities until it reaches the final destruction facility. This includes all stops where a change in transporter occurs. Accessibility to the property while in temporary storage shall be restricted to prime Contractor personnel. This CLIN shall be ordered by the Government even when the property is deemed non-RCRA hazardous. The Contractor shall provide a CD to the Government within 30 calendar days following destruction.

b. Generators turning in property assigned CLIN TX99 are certifying acknowledgement of these terms and conditions, and are responsible for obtaining proper authorization from their chain of command prior to turn-in.

2.5.2 OTHER MILITARY ITEMS

a. Military waste items are assigned CLINs according to the physical and chemical properties of the waste.

b. RCRA Hazardous Military Items – When a military item meets the definition of RCRA hazardous waste, a CLIN may be assigned to the waste similarly as other hazardous wastes, e.g., flammable debris, toxic solid, etc.  Other than CLIN TX99, military items that are deemed RCRA HW do not require a CLIN that is uniquely assigned to military items requiring DEMIL.  In some cases, for DEMIL requirements, the Government may require a CD for specific RCRA-hazardous military items.  DEMIL-specific CLINs with a CD requirement will be used for these items.

c. Under this contract, non-RCRA military items may be turned in under Base CLIN NR97. This CLIN requires RCRA incineration and the Contractor’s provision of a CD.

3.0 SPECIAL SERVICES

Special services under this contract are intended to facilitate waste removals and ensure proper DOT or TSDF packaging requirements. These services will be ordered in conjunction with related disposal CLINs.

3.1 PERFORM OVERPACKING OR LOOSE PACKING SERVICES – CLINs 1202SV-1207SV

The Government may order special services for the Contractor to perform overpacking or loose packing of containers for such reasons as containing a leaking container, precautionary packaging to prevent leakage, packaging multiple containers into a single container, ensuring proper packaging for DOT shipping or TSDF acceptance, and for convenience in general.  While overpacking and loose packing serve different purposes, both require the placement of smaller container(s) into a single larger container.  As such, both services are provided under common CLINs based on the outer container sizes.  Outer containers are provided by the Government.

a. An overpack drum may be used as a precautionary measure to preemptively contain waste, should the primary container(s) pose potential to leak.  However, a salvage drum will be used for packaging leaking, damaged, or defective container(s) per 49 CFR 173.3(c).  The salvage drum must be compatible with the lading in the leaking/damaged package(s).  When overpacking of container(s) is ordered by the Government under CLINs 1202SV-1206SV, the Contractor is responsible for utilizing an appropriate container size, type, materials, markings, and labeling.  Inner container(s) shall be kept in an upright position when practicable.  Overpack and salvage drums that are used for liquids shall be marked with directional arrows indicating the orientation of the inner container(s).

b. A loose pack container is an overpack of smaller containers, typically consumer commodity products of the same type, e.g., paints, lubricants, adhesives, expired products, etc. and provides the best alternative when consolidation (e.g., pouring) is impractical.  The loose pack container shall be of the appropriate UN rating for the lading.  When loose packing of containers is ordered by the Government under CLINs 1202SV-1207SV, the Contractor is responsible for ensuring the appropriate container type, size, materials, markings, and labeling are utilized.

c. The Contractor shall utilize an appropriate container purchased and provided by the Government.  If the Government does not possess an appropriate container for the waste(s) being packed, the Government shall remove the subject special services and disposal CLINs from the Task Order.

d. Services provided under CLINs 1202SV-1207SV do not include transportation or disposal of the resulting containers.  Removal and disposal of overpack and loose pack containers shall be separately ordered under the appropriate disposal CLINs on the same TO.

e. The following table summarizes the various overpacking and loose packing services:

| CLIN | Overpacking and Loose Packing Special Services Descriptions |
| --- | --- |
| 1202SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Government-provided overpack container up to five (5) gallons in size. |
| 1203SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Government-provided overpack container up to 15 gallons in size. |
| 1204SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Government-provided overpack container up to 30 gallons in size. |
| 1205SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Government-provided overpack container up to 55 gallons in size. |
| 1206SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to overpack or loose pack one or more smaller containers into one (1) Government-provided overpack container up to 110 gallons in size. |
| 1207SV | One (1) EA shall include all labor, tools, equipment, labels, sorbent, etc. needed to loose pack smaller containers into one (1) Government-provided cubic yard box/military tri-wall box. |

3.2 PERFORM DDR LITHIUM BATTERY PACKAGING SERVICES – CLIN 1221SV

a. Damaged, Defective, or Recalled (DDR) lithium batteries can pose a significant hazard while in storage or transport.  As such, these compromised batteries must be packaged to prevent short circuiting, ignition, and chemical reaction that could lead to fire.  The Government may order Contractor packaging services for DDR lithium batteries to ensure compliant DOT packaging prior to shipment.

b. When CLIN 1221SV is ordered, the Contractor shall prepare and package for transporting DDR lithium batteries IAW 49 CFR §173.185(f) or updates thereto. See https:/​/​www.ecfr.gov/​current/​title-49/​part-173/​section-173.185#p-173.185(f) for details. The following is provided as guidance only and shall not be construed as regulatory compliant.

(1) Each loose battery must be placed in an individual, non-metallic inner package that completely encloses the battery.  A conglomerate comprised of multiple melted, fused, or affixed batteries will be packaged as one, placed in a non-metallic inner package that completely encloses the battery mass.

(2) Inner packages will be surrounded by cushioning material that is non-combustible, electrically non-conductive, and absorbent.

(3) Inner packages must be further packaged in an outer container meeting the requirements of Part 178, Subparts L, M, P, or Q (Packing Group I), or updates thereto.

(4) The outer container must be marked to indicate the package contains a “Damaged/Defective Lithium-Ion Battery” or “Damaged/Defective Lithium Metal Battery”.

c. Prior to packaging, the batteries will be staged in a Government-provided workspace that is easily accessible to the Contractor.

d. The Contractor is authorized to perform these packaging services on the same day as removal or during a pre-removal site visit at the Contractor’s discretion.  A Generator representative will provide Government oversight of the Contractor’s performance.

e. CLIN selection for the packaging service will be based on the outer container size needed to appropriately complete the packaging.  CLIN selection and container size will be mutually agreed upon prior to commencement of work.  Packaging services provided under CLINs 1221SV-1224SV do not include transportation or disposal of the resulting containers.  Removal and disposal of the package(s) shall be ordered separately under CLIN BA17C7 on the same TO. See 2.1.

f. The following table summarizes the various DDR Lithium Battery Packaging Services:

| CLIN | DDR Lithium Battery Packaging Services |
| --- | --- |
| 1221SV | One (1) EA shall include all labor and supplies needed to properly package and label one DDR lithium battery or battery series IAW current DOT regulations into one outer container up to five (5) gallons in size. |
| 1222SV | RESERVED for outer containers up to 15 gallons in size, if added to the contract. |
| 1223SV | RESERVED for outer containers up to 30 gallons in size, if added to the contract. |
| 1224SV | RESERVED for outer containers up to 55 gallons in size, if added to the contract. |

3.3 PROVIDE CERTIFICATE OF DISPOSAL OR DESTRUCTION (CD) – CLIN 1300SV

a. Generators may order Certificates of Disposal or Destruction (CD) for any items at their discretion.  The Government recognizes there are additional costs to the Contractor associated with preparing these certificates.  Provision of CDs will only be required as specified in the CLIN description on the Price Schedule and/or PWS, or if CLIN 1300SV is ordered.  CLIN 1300SV will not be ordered in conjunction with a CLIN that is already described in the Price Schedule as requiring a CD.  Example: CLIN 1300SV will not be ordered to provide the CD for a CLIN that is greater than 50 ppm PCB because contract Section 2.2 already requires the Contractor to provide CDs for all PCB-related waste containing, or contaminated by, a source that is equal to or greater than 50 ppm PCBs (or 5 ppm in California and 2 ppm in Washington State).

b. CLIN 1300SV will be ordered as one each (1 EA) for every Task Order Line Item requiring a CD.

c. CDs shall clearly indicate, or be supplemented with supporting documentation to clearly indicate, a complete “cradle-to-grave” audit trail from the Generator pickup point to the final incineration or disposal facility.  This audit trail will require, at a minimum, quantities, TO, EPA ID number(s), manifest(s) and line-item number(s) that serve to clearly link the initial removal shipping document, through all interim facilities as applicable, to the final incineration or disposal facility that is issuing the certificate.  A data spreadsheet may be used by the Contractor (in lieu of manifest copies) to accompany a CD to provide the minimally required cradle-to-grave information.  Such information provided by the Contractor is subject to verification via Government audit of Contractor and TSDF manifest files.

d. Some facilities issue CDs that, in essence, state the waste “will be” versus “has been” disposed or destroyed IAW applicable regulations.  Such certificates will be authorized under this Section, but not for DEMIL-required items, and only when issued and certified by the final incineration or disposal facility.

e. CDs shall be provided to the Generator and the COR within 30 days after destruction or disposal occurs.  The Contractor will not be authorized payment until all required CDs are received by the COR and Generator that ordered the certification.

3.4 PROVIDE HAZARDOUS WASTE PROFILE SHEET (HWPS) - CLIN 1301SV

a. When CLIN 1301SV is ordered, the Contractor is required to prepare and provide a typed Hazardous Waste Profile Sheet (HWPS), DLA Form 2511, even when the subject waste does not meet the EPA definition of HW.  One each (1 EA) will be ordered per waste stream being profiled.  Using analytical data and/or information provided by the Generator, the Contractor will complete the form IAW attached instructions.  Subject to COR approval, the Contractor may use an equivalent waste profile form, provided the alternate form contains the same essential information contained in the DLA Form 2511.

b. The purpose of this service is to enable the Generator to turn in items to the Contractor for removal.  It is not intended to supplant Contractor obligations identified in other Sections of this contract.

4.0 ACCEPTABLE PERFORMANCE LEVEL (APL)

a. Acceptable Performance Level (APL) is the minimum performance of each requirement before the Government considers performance unsatisfactory.  Specification of an acceptable performance level does not allow the Contractor to knowingly provide defective service; it is recognition of the fact that defective performance may sometimes occur unintentionally.  If the performance percentage does not fall below the specified APL, the Government will not deduct for poor performance.  However, the Contractor shall be required to re-perform or correct the defective service or product at no additional cost to the Government.

b. The surveillance method shall be from Collection Summary Reports (CSR) furnished by the COR and tracking of final destruction/disposal submittals.  All negative and positive actions/performance shall be the basis for monetary deductions and/or favorable or unfavorable past performance annotations.

c. Performance incentives for the Contractor shall include but are not necessarily limited to favorable impact on option exercise decisions, future awards of the same requirement, and future awards of similar requirements by DLADS as well as other Government and non-Government entities that may review performance of this contract.

4.1 ACCEPTABLE PERFORMANCE LEVEL (APL) TABLE

| DELIVERABLE | STANDARD | TIME FRAME | Including, but not restricted to Section | ACCEPTABLE PERFORMANCE LEVEL (APL) | METHODS OF SURVEILLANCE |
| --- | --- | --- | --- | --- | --- |
| Timely TO Performance | Timely performance of TO. | As stated in the contract for service specified. | 0.0, 1.1, 1.2, 3.0, 7.0, and other Sections throughout the PWS. | No more than 6 occurrences of non-timely performance during a 12-month period. | 100% Review of Collection Summary Reports (CSRs). |
| Compliant Pre-Removal | All Shipping documentation (including manifests and LDRs), provided by the Contractor were correct the first time, in accordance with the information on the TO and copies sent to the COR for review within the allotted time. | As stated in contract. | 1.1.3 and other Sections throughout the PWS. | No more than 6 occurrences where submissions were returned for missing or incorrect data and no more than 6 occurrences for late submissions during a 12-month period. | 100% Review of Collection Summary Reports (CSRs). |
| Compliant Removal | The Contractor arrived on time and with the required equipment in good working order to safely complete the removal or service. The Contractor personnel were qualified, and names were provided to COR ≥ 24 hours prior to removal. | As stated in contract. | 1.1.4, 1.1.5, 1.1.10, 1.2.6, 1.2.7, and other Sections throughout the PWS. | No more than 6 occurrences where Contractor did not meet these requirements during a 12-month period. | 100% Review of Collection Summary Reports (CSRs). |
| Compliant Post-Removal | The Contractor provided all required documents as outlined within the contract PWS and within the required timelines to the COR (e.g., all shipping documents, return manifests, and CDs). | As stated in contract. | 1.3.1, 8.1, 8.2, and other areas throughout the PWS. | No more than 6 occurrences where submissions were returned for missing or incorrect data and no more than 6 occurrences for late submissions during a 12-month period. | 100% Review of Collection Summary Reports (CSRs). |

The Government reserves the right to conduct additional surveillance/review of final tracking information and/or delay payments until after complete and accurate tracking information is received, as warranted, when performance problems occur in this area.

5.0 QUALIFIED FACILITIES AND TRANSPORTERS LISTS (QFL/QTL)

a. The Contractor will only utilize facilities and transporters listed on the Qualified Facilities List (QFL) and Qualified Transporters List (QTL) when performing removals for disposal services.  The QFL and QTL is located on the DLA Disposition Services Website at:  https://www.dla.mil/DispositionServices/Offers/Disposal/HazardousWaste/HazWasteDisposal.aspx.

b. The Contractor is responsible to ensure their services and Subcontractor services are performed IAW:

- The contract terms and conditions, and

- All federal, state, and local laws and regulations.

c. When a firm is listed on the QFL and QTL, DLA Disposition Services is not declaring that the facility and transporter meet the requirements of this contract or that the facility and transporter provides services IAW all applicable laws, regulations, and standards.  COR approval of shipping paperwork also does not absolve the Contractor from ensuring their services or Subcontractor services meet these requirements.  If firms listed on the QFL or QTL do not meet contract terms and conditions or are unable to perform IAW all applicable laws, regulations, and standards, the Contractor will propose additional firms that meet these requirements.

d. Any damages, spills, contamination, violations and/or fines resulting from Contractor or Subcontractor provided services will be the responsibility of the Contractor to correct such issues to the satisfaction of the Government at no additional cost to the Government.  The Contractor will provide the Government with proposed corrective actions and the Government will determine if the corrective actions are sufficient.  The Contractor will reimburse the Government for any fines that result from the Contractor or Subcontractor services.  The Contractor will also reimburse the Government for paid services that result in damages, spills, contamination, violations and/or fines.

5.1 ADDING/UPDATING QUALIFIED FACILITIES AND TRANSPORTERS

a. TOs bearing disposal CLINs that haven’t been assigned in the Hazardous Material Management System (HMMS) to their designated TSDF EPA ID Number cannot be issued until DLA Disposition Services has completed the assignments for those CLINs going to that TSDF. Most well-established TSDFs in the U.S. have associated CLINs already assigned them through other DLA contracts.  However, when a new or previously unused TSDF is added to the QFL, CLINs that are destined for that TSDF must first be assigned to the TSDF in HMMS by DLA Disposition Services. To accomplish this, the Contractor will complete a Facility-CLIN Assignment Sheet referencing potential Base CLINs destined for the TSDF Name and EPA ID# during the life of the contract. The Contractor will email the completed Facility-CLIN Assignment Sheet to TSDFandTransporterInquiries@dla.mil and cc: the KO.  This is only required when the designated TSDF has never received these CLINs in the past.  When the Contractor requests the addition or update of a qualified facility, the Contractor will indicate all disposal Base CLINs to the QFL Application or QFL Update form, respectively.  CLINs may be added to a qualified facility anytime during the life of the contract, provided the CLIN descriptions including permissible EPA Waste Codes are compliant with the facility’s RCRA Permit.

b. The Contractor will review the QFL and QTL prior to each removal to ensure the facilities and transporters are listed on the QFL and QTL and have the appropriate permits to perform the services they provide.

c. The awarded prime Contractor will be the only entity to request additions and updates to the QFL and QTL.

d. Contractor inquiries and applications for additions and updates will be sent to the following email address with a cc: to your KO and COR:  TSDFandTransporterInquiries@dla.mil.

e. The Contractor will complete DLA Form 2507, Application for Qualified Facility List (QFL) to add new or update existing facilities. The most current version of this form is available at https://www.dla.mil/Forms/.

f. The Contractor will complete DLA Form 2503, Application for Qualified Transporter List (QTL) to add new or update existing transporters. The most current version of this form is available at https://www.dla.mil/Forms/.

g. The proposed facility or transporter will not be added or updated to the QFL and QTL if requested documentation is not provided or if any of the reasons in section 5.2 through 5.3 apply.

h. The request by the Contractor to have a facility or transporter added to the QFL and QTL after contract award does not relieve the Contractor of their obligation to perform IAW existing CLIN prices.

i. The rejection/removal of a facility or transporter from the QFL/QTL does not relieve the Contractor from performing all contract requirements IAW existing CLIN prices.

5.2 FACILITY REMOVAL

At any time during the period of this contract, the Government will remove a facility from the QFL if any of the following applies:

a. The facility is no longer in operation.

b. The facility is a significant non-complier (including, but not limited to, RCRA Class 1 violations) and has not entered into a compliance schedule or similar action.

c. The facility received an administrative order or judicial action and has not entered into a compliance schedule or similar action within 180 days of the time of issued order or judicial action.

d. The facility has a history of non-compliance (including but not limited to non-conformance, RCRA class I and II violations, OSHA violations, state and local violations, etc.) or exhibits a lack of "good faith" in correcting violations.  A "good faith" effort is promptly signing a consent agreement with the regulatory authorities and performing in compliance with the agreement for at least six months.  Repeated violations indicate a lack of "good faith".

e. The facility has groundwater contamination or does not meet groundwater anti-degradation policy.

f. The facility does not have adequate permits or is not capable of handling the proposed waste.

g. The facility received a negative recommendation resulting from a DLA Disposition Services site visit or a third-party audit conducted on DLA Disposition Services’ behalf without substantive evidence of corrected deficiencies.

h. The facility transports DoW waste to a facility not on the QFL.

i. The facility's financial assurance is not sufficient to protect the Government's long-term interests.

j. The facility is unable to demonstrate tracking of DoW waste from entry to exit.

k. The facility manages property in a manner that causes the generator of the manifest to file exception reports IAW 40 CFR 262.42 or state/host nation equivalent.

l. The facility has not received DLA Disposition Services waste for a period of 12 consecutive months.

m. Requested facility documentation is not provided.

n. The facility operates in a manner that is not IAW this contract.

o. The facility does not have or does not enforce 24-hour security measures.

p. The facility does not allow or limits access for a site visit by DLA Disposition Services personnel or representatives.

q. The facility caused damages, spills, contamination, and/or fines while providing services for this contract.

r. The Government reserves the right to disapprove any facility when it is in the public interest to do so.

5.3 TRANSPORTER REMOVAL

At any time during the period of this contract, the Government will remove a transporter from the QTL if any of the following applies:

a. The transporter does not have the appropriate hazardous or non-hazardous Federal/state/local/host nation permits to transport property under this contract.

b. Transporter has an "unsatisfactory" or “conditional” rating from DOT Office of Motor Carriers (OMC).  For OCONUS, the transporter has exhibited a history of noncompliance.

c. The transporter has exhibited a history of acute/critical violations in the Federal Motor Carrier Safety Administration (FMCSA) Safety Management System (SMS) and there is no get-well plan or the get-well plan milestones have been missed.

d. The transporter received an administrative order or judicial action and has not entered into a compliance schedule or similar action within 180 days from the time the order or issued judicial action.

e. The transporter does not have or does not enforce 24-hour security measures.

f. The transporter manages property in a manner that causes the generator of the manifest to file exception reports IAW 40 CFR 262.42 or state/host nation equivalent.

g. Transporter transfer locations are unable to track property from entry to exit.

h. The transporter has not shipped DLA Disposition Services wastes for a period of twelve consecutive months.

i. The transporter co-mingles DOW and non-DOW waste prior to receipt at the manifested facility.

j. The transporter operates in a manner that is not IAW this contract.

k. Requested transport documentation is not provided.

l. The transporter caused damages, spills, contamination, and/or fines while providing services for this contract.

m. The Government reserves the right to disapprove of any transporter when it is in the public interest to do so.

6.0 ADDITIONAL REQUIREMENTS

6.1 INDEMNIFICATION

a. Upon receipt/removal of items from the various Government installations, the Contractor assumes full accountability and physical custody of such items.

b. DLA, its officials, agents, Contractors, Subcontractors, employees, and representatives, shall be held harmless from all claims or causes of action arising from, or on account of, negligent or other wrongful acts, omissions of the Contractor, its officers, directors, employees, agents, or Subcontractors, in performing the Work pursuant to this contract.  In addition, the Contractor agrees to pay the United States all costs incurred by the United States, including but not limited to attorney’s fees and other expenses of litigation and settlement, arising from or on account of claims made against the United States based on negligent or other wrongful acts or omissions of the Contractor, its officers, directors, employees, agents, subcontractors and any persons acting on its behalf or under its control, in performing the Work pursuant to this contract.

c. The Contractor also agrees to hold the Government harmless and indemnify the Government for any and all costs, including those that arise from violation of any law, regulation, order, right, or public or private enforcement program under which the government incurs environmental clean-up or response costs, judgments, action, debt, liability costs and attorney’s fees or any other requests for monies or any other type of relief arising from or incident to the work performed pursuant to this contract.

d. The United States shall not be held out as a party to any contract entered into by or on behalf of the Contractor in performing the Work pursuant to this Contract.  Neither Contractor nor any such Subcontractors shall be considered an agent of the United States.

e. The United States shall give Contractor notice of any claim for which the United States plans to seek indemnification pursuant to this Section and shall consult with Contractor prior to settling such claim.

6.2 PACKAGING, MARKING, LABELING, AND PLACARDING

- The Contractor shall verify before transporting that all containers are properly packaged, labeled, marked, and placarded to meet all applicable U.S. DOT and applicable State regulations.  The Contractor shall provide and affix the appropriate placards to each conveyance vehicle and trailer prior to leaving Government premises.

- The Contractor shall not package RCRA-regulated or State-regulated waste together with non-regulated waste.

6.3 PLACES OF GOVERNMENT INSPECTION

- All services will always be subject to inspection by the KO and his/her authorized representative(s) to include Government contracted audit companies.  The Government and/or its representatives shall have the right to inspect and obtain copies of all written licenses, permits, and approvals issued by any Governmental entity or agency to the Contractor or its Subcontractors which are applicable to the performance of services under this contract; to inspect and test, at its own expense, transportation vehicles or vessels, containers, and disposal facilities provided by the Contractor; and to inspect the handling, loading, transportation, storage, and disposal operations conducted by the Contractor or its Subcontractors in the performance of this contract.

- The Government and/or its representatives shall be offered free access to any facility used by the Contractor and any Subcontractors in performing services under this contract, including offices and facilities where contract-related records are retained.  Government inspections of Contractor facilities may be scheduled or unscheduled, i.e., announced, or unannounced.  The purpose of these inspections is to assist the Government in determining the conformance of services with contract requirements.

- The Contractor is solely and exclusively responsible for the quality of all services performed under this contract.  The Government's right to conduct inspections at Government, Contractor, or Subcontractor facilities does not relieve the Contractor of this responsibility.  Neither Government failure to make such inspection, nor failure to discover nonconforming services, shall prejudice the rights of the Government thereafter to reject services, nor relieve the Contractor of its obligation to perform work strictly in accordance with the contract.

- The Contractor, in its agreements with Subcontractors, shall ensure that the inspection rights described herein are afforded the Government and/or its representatives by each Subcontractor performing services under this contract.

- As part of the U.S. Government’s right to conduct inspection of services under this contract, it shall be allowed to obtain information from regulators that may be relevant to assessing the Contractor’s historical or current compliance status with environmental law and regulations.

6.4 CONTRACTOR REPRESENTATIVE

The Contractor shall provide in the space below, the name and telephone number of at least one responsible individual who will serve to respond to operational problems and emergencies on a 24-hour, seven (7) days/week (or “24/7”) basis.

The Contractor agrees that notice to the Contractor representative(s) listed constitutes notice to the Contractor and agrees to be bound by any commitments or representations made by this representative.

NAME:

TITLE:

PHONE NUMBER:

24 HOUR EMERGENCY NUMBER:

6.5 INCIDENTAL FEES

Any incidental state or local fee, tax or penalty assessed against DOW/DLADS arising from the Contractor’s decisions in the performance of this contract will be paid by the Contractor, including, but not limited to: Any fee, tax, or penalty levied because of a contractor’s failure to prepare and provide reports and/or documents required as part of the performance of this contract.

7.0 STANDARD PERFORMANCE TIME FOR WASTE REMOVALS AND PROVISION OF OTHER SERVICES

a. The standard time for removal of containerized waste and provision of other services including overpacking and loose packing under this contract, is 30 business days.

b. The clock for performance of waste removals and other services begins on the first full business day following issuance of the written Task Order.  All waste removals and other services shall be completed by close of business on the last day of the required removal time.

8.0 DOCUMENTATION REQUIREMENTS, DLA DISPOSITION SERVICES ACCEPTANCE, INVOICING, AND TRACKING

8.1 SUBMISSION OF DOCUMENTATION TO FACILITATE ACCEPTANCE FOR PAYMENT

a. The Contractor shall submit one invoicing packet, via email, for all line items listed on the Task Order (TO) after waste has been received at the first TSDF as designated on the pickup manifest, and services have been completed except for waste requiring a CD which must therefore be tracked to the final destruction or disposal facility.

b. PRIOR to submission of invoicing packet, the Contractor shall coordinate with the COR to ensure:

(1) Return copies of all shipping papers have been received by the COR.

(2) All modifications, changes, certifications, e.g., certification of destruction/disposal, and/or updates have been completed and/or received by the Government.

c. An invoice packet shall consist of:

(1) Invoice (DLA Form 2505)

(a) Must use the most up-to-date official form which can be located at: https://www.dla.mil/Forms/DLFormList/.

(b) Must be in Line-Item order.

(c) All header fields and columns 1 through 14 must be completed.

(d) Must reflect all changes on MOD paperwork.

(2) Pick-up Report (DD Form 1155)

(a) A short-form Business Objects report may be used in lieu of the original Pick-up Report provided it includes the 1st and 2nd pages of the original DD form 1155.

(b) Must be in Line-Item order.

(c) All data fields must be completed, and all applicable signature fields signed.

(d) Must be legible and accurate.

(e) Must provide full manifest/document number.

(f) Signed copies of all modifications (if applicable).

(3) All return shipping documents, which include but are not limited to, Hazardous Waste Manifests and Non-Hazardous Waste Manifests, must be complete, accurate and legible, and will include:

(a) Correct EPA ID number(s) for the Generator, Transporter, and the TSDF (or a Mock EPA ID number when no EPA ID number exists).

(b) The contract and Task Order number.

(c) Signatures and dates from the Generator, the transporter, and the accepting facility.

d. The invoicing packet shall be submitted to DLADispSvcsManifestTracking@dla.mil using the following guidance:

(1) Subject line shall be formatted Contract Number/Task Order Number, Company Name (e.g., SP450017D0012/18F0012, Waste Pickup Co.).

(2) Attachments shall be named Contract Number/Task Order Number (e.g., SP450017D0012/18F0012).

(3) ONLY packets that exceed 10MB in size are authorized to be split into separate emails using the following guidance:

(a) Emails shall be numbered at the end of the subject line to identify a multiple email submission (e.g., SP450017D0012/18F0012, Waste Pickup Co. 1/3).

(b) Attachment names shall receive a lowercase letter as a suffix to identify a multiple attachment packet (e.g., SP450017D0012/18F0012a).

(4) Each email submitted shall contain only one file.

e. Packets received by the tracking department that do not meet these requirements will be rejected.

(1) Rejected packets must be resubmitted in their entirety.

(2) Reviews will not be continued after the first discrepancy is identified.

8.2 ACCEPTANCE AND INVOICING

a. Except for wastes requiring a CD, all disposal CLINs are considered disposed on the date the items are accepted by the first, properly permitted, qualified TSDF.  For wastes requiring a CD, for purposes of payment, disposal/demilitarization for all property is considered completed when final disposal/demilitarization has occurred. For service “1000, 1200, and 1300 Series” CLINs, the services are considered complete on the date the services are completed delivered.  For service CLIN 1300SV, the service will be considered complete once the Government receives the ordered certification(s) from the Contractor.  The Government period for acceptance of the invoice package is within 20 calendar days of receipt of the Contractor’s acceptable invoice packet that provides final Government acceptance of the services performed.  The contractor will be paid using the Evaluated Receipt Settlement (ERS) process when their manifest tracking packet is authorized for payment in the Hazardous Material Management System (HMMS). Payment to the Contractor will be made through Electronic Funds Transfer (EFT) in accordance with the Prompt Payment Act.

b. Failure to submit timely and complete documentation for acceptance of payment on a Task Order will result in late payment or non-payment for services rendered.  Payment may be approved upon resubmission of the correct documentation for all line items on the Task Order.  The Government can, at the discretion of the KO, utilize other remedies including those indicated below.

c. Note that consideration pursuant to FAR 52.212-4(a) can be taken against any open Task Order under the contract, not necessarily the Task Order with documented deficiencies.  In addition, consideration may also be taken against another contract if there are no open task orders on the subject contract.

d. By submitting for payment in accordance with the above terms including any post-payment documentation requirements, the Contractor certifies that all services, including receipt at initial qualified TSDF and ultimate disposition of the items, conform in all respects with contract requirements.

e. The Contractor shall comply with the requirements of FAR Subpart 42.12 regarding a company change-of-name and agree in writing to the timeline and procedures specified by the responsible KO.  The Contractor must provide the KO with documentation to support the legally changed company name.

f. For any discrepancies in payment, contact j8bvendorpayments@dla.mil.

9.0 CONTRACT MINIMUM/MAXIMUM

This is a firm-fixed price indefinite quantity contract. The contract minimum shall be $25,000.00.  The maximum shall be 125% of the estimated value of the contract.  There is no contract minimum for any option period if included in the contract.

10.0 CONTRACTOR ACCESS TO DLA DISPOSITION SERVICES SITES

Throughout the life of the contract, the KO may authorize the Contractor to obtain a Common Access Card (CAC) to perform work under this contract.  The Contractor shall provide justification for the CAC to the KO.  If a CAC is authorized, DLA Form 1728, DD Form 1172-2, CAC Justification Form, and OF306 will be submitted to the KO for review and approval.  If authorized the issuance of a CAC, the Contractor is required to follow procedures identified in section 10.2.

10.1 CONTRACTOR PERSONNEL SECURITY REQUIREMENTS (H.14)

a. Work to be performed under this contract or task order may, in full or in part, be performed at the DLA Headquarters (HQ), DLA field activity office(s), or other Federally controlled facilities.  Prior to beginning work on a contract, DLA requires all Contractor personnel working on the Federally controlled facility to have at a minimum, an initiated National Agency Check with Written Inquiries (NACI) or NACI equivalent and favorable completion of a Federal Bureau of Investigation (FBI) fingerprint check.

b. Additionally, in accordance with DoD Regulation 5200.2-R, Personnel Security Programs, and DLA Issuance 4314, Personnel Security Program, all DoD contractor personnel who have access to Federally controlled information systems must be assigned to positions which are designated at one of three information technology (IT) levels, each requiring a certain level of investigation and clearance, as follows:

(1) IT-I for an IT position requiring a single scope background investigation (SSBI) or SSBI equivalent,

(2) IT-II for an IT position requiring a National Agency check with Law and Credit (NACLC) or NACLC equivalent, and

(3) IT-III for an IT position requiring a NACI or equivalent.

Note: IT levels will be designated according to the criteria in DoD 5200.2-R.

c. Previously completed security investigations may be accepted by the Government in lieu of new investigations if determined by the DLA Intelligence Personnel Security Office to be essentially equivalent in scope to the contract requirements.  The length of time elapsed since the previous investigation will also be considered in determining whether a new investigation is warranted.  To assist the Government in making this determination, the contractor must provide the following information to the respective DLA Intelligence Personnel Security Office immediately upon receipt of the contract.  This information must be provided for each contractor employee who will perform work on a Federally controlled facility and/or will require access to Federally controlled information systems:

(1) Full name including middle name as applicable, with social security number.

(2) Citizenship status with date and place of birth.

(3) Proof of the individual’s favorably adjudicated background investigation or NACI, consisting of identification of the type of investigation performed, date of the favorable adjudication, name of the agency that made the favorable adjudication, and name of the agency that performed the investigation.

(4) Company name, address, phone and fax numbers with email address.

(5) Location of on-site workstation or phone number if off-site (if known by the time of award).

(6) Delivery order or contract number and expiration date; and name of the contracting officer.

d. The KO will ensure that the contractor is notified as soon as a determination is made by the assigned or cognizant DLA Intelligence Personnel Security Office regarding acceptance of the previous investigation and clearance level.

(1) If a new investigation is deemed necessary, the contractor and contracting officer KO will be notified by the respective DLA Personnel Security Office after appropriate checks in DoD databases have been made.

(2) If the contractor employee requires access to classified information and currently does not have the appropriate clearance level and/or an active security clearance, the DLA Intelligence Personnel Security Office will relay this information to the contractor and KO for further action.  Investigations for contractor employees requiring access to classified information must be initiated by the contractor Facility Security Officer (FSO).

(3) The KO will ensure that the respective DLA Intelligence Personnel Security Office initiates investigations for contractor employees not requiring access to classified information (i.e., IT or unescorted entry).

(4) In order to begin the investigation process for the required clearance level it is the contractor’s responsibility to ensure that adequate information is provided and that each contractor employee completes the appropriate paperwork, as required either by the KO or the DLA Intelligence Personnel Security Office,

e. The contractor is responsible for ensuring that each contractor employee assigned to the position has the appropriate security clearance level.

f. The contractor shall submit each request for IT access and investigation through the KO to the assigned DLA Intelligence Personnel Security Office.  Requests shall include the following information and/or documentation:

(1) Standard Form (SF) 85, Questionnaire for Non-Sensitive Positions, or SF 86, Questionnaire for National Security Positions (see note below),

(2) Proof of citizenship (i.e., an original or a certified copy of a birth certificate, passport, or naturalization certificate), and

(3) Form FD-258, Fingerprint Card (however, fingerprinting can be performed by the cognizant DLA Intelligence Personnel Security Office).

(Note to 6.a above: An investigation request is facilitated through use of a SF 85 or the SF 86.  These forms with instructions as well as the Optional Form (OF) 306, Declaration for Federal Employment, which is required with submission of the SF 85 or SF 86, are available at the Office of Personnel Management’s (OPM) system called Electronic – Questionnaires for Investigations Processing (e-QIP). Hard copies of the SF 85 and SF 86 are available at OPM’s website, www.opm.gov, but hard copies of the forms are not accepted.)

g. Required documentation, listed above in paragraphs 6.a through 6.c, must be provided by the contractor as directed by the KO to the cognizant DLA Intelligence Personnel Security Office at the time of fingerprinting or prior to the DLA Intelligence Personnel Security Office releasing the investigation to OPM.

h. Upon completion of the NACI, NACLC, SSBI, or other sufficient, appropriate investigation, the results of the investigation will be forwarded by OPM to the appropriate adjudication facility for eligibility determination or the DLA Intelligence Personnel Security Office for review and determination regarding the applicant’s suitability to occupy an unescorted entry position in performance of the DLA contract.  Contractor personnel shall not commence work on this effort until the investigation has been favorably adjudicated, or the contractor employee has been waived into the position pending completion of adjudication.  The DLA Intelligence Personnel Security Office will ensure that results of investigations will be sent by OPM to the DoD, Consolidated Adjudications Facility (DoDCAF) or DLA Intelligence Personnel Security Office.

i. A waiver for IT level positions to allow assignment of an individual contractor employee to commence work prior to completion of the investigation may be granted in emergency situations when it is determined that a delay would be harmful to national security.  A request for waiver will be considered only after the Government is in receipt of the individual contractor employee’s completed forms, the background investigation has been initiated, and favorable FBI fingerprint check has been conducted.  The request for a waiver must be approved by the Commander/Director or Deputy Commander/Director of the site.  The cognizant DLA Intelligence Personnel Security Office reserves the right to determine whether a waiver request will be forwarded for processing.  The individual contractor employee for which the waiver is being requested may not be assigned to a position, that is, physically work at the Federally controlled facility and/or be granted access to Federally controlled information systems, until the waiver has been approved.

j. The requirements of this procurement note apply to the prime contractor and any subcontractors the prime contractor may employ during this contract, as well as any temporary employees that may be hired by the contractor.  The Government retains the right to request removal of contractor personnel, regardless of prior clearance or adjudication status whose actions, while assigned to this contract, are determined by the KO to conflict with the interests of the Government.  If such removal occurs, the contractor shall assign qualified personnel, with the required investigation, to any vacancy.

k. All contractor personnel who are granted access to Government and/or Federally controlled information systems shall observe all local automated information system (AIS) security policies and procedures.  Violations of local AIS security policy, such as password sharing, performing personal work, file access violations, or browsing files outside the scope of the contract, will result in removal of the contractor employees from Government property and referral to the contractor for appropriate disciplinary action.  Actions taken by the contractor in response to a violation will be evaluated and will be reflected in the contractor’s performance assessment for use in making future source selection decisions.  In addition, based on the nature and extent of any violations of AIS security policy, the Government will consider whether it needs to pursue any other actions under the contract such as possible termination.

l. The contractor may also be required to obtain a CAC or Installation Access Badge for each contractor employee in accordance with procedures established by DLA.  When a CAC is required, the KO will ensure that the contractor meets the requirements of Homeland Security Presidential Directive 12 and any other CAC-related requirements in the contract.  The contractor shall provide, monthly, a listing of all personnel working under the contract that are in possession of a CAC.

m. Contractor personnel must additionally receive operations security (OPSEC) and information security (INFOSEC) awareness training.  The DLA annual OPSEC refresher training and DLA annual INFOSEC training will satisfy these requirements and are available through the DLA Intelligence Office.

n. When the contractor employee who has been granted a clearance is removed from the contract, the contractor shall provide an appropriately trained substitute who has met or will meet the investigative requirements of this procurement note.  The substitute may not begin work on the contract without written documentation, signed by the KO, stating that the new contractor employee has met one of the criteria set forth in paragraphs (c), (d), or (i) of this procurement note (i.e., acceptance of a previously completed security investigation, satisfactory completion of a new investigation, or a waiver allowing work to begin pending completion of an investigation).  Contractor individual employees who are removed from this contract because of a violation of local AIS security policy are removed for the duration of the contract.

o. The following shall be completed for every employee of the Government contractor working on this contract upon contract expiration.  Additionally, the contractor shall notify the KO immediately in writing whenever a contractor employee working on this contract resigns, is reassigned, is terminated, or no longer requires admittance to the Federally controlled facility or access to Federally controlled information systems.  When the contractor employee departs, the contractor will relay departure information to the cognizant DLA Intelligence Personnel Security Office and the Trusted Agent (TA) that entered the individual into the Trusted Associated Sponsorship System (TASS), so appropriate databases can be updated.  The contractor will ensure each departed employee has completed the DLA J6 Out-Processing Checklist, when applicable, for the necessary security briefing, has returned any Government furnished equipment, returned the DoD CAC and DLA (or equivalent Installation) badge, returned any DoD, DoW, or DLA vehicle decal, and requested deletion of local area network account with a prepared DoD (DD) Form 2875.  The contractor will be responsible for any costs involved for failure to complete the out-processing, including recovery of Government property and investigation involved.

p. These contractor security requirements do not excuse the contractor from meeting the delivery schedule/performance requirements set forth in the contract or wave the delivery schedule/performance requirements in any way.  The contractor shall meet the required delivery schedule/performance requirements unless the KO grants a waiver or extension.

q. The contractor shall not bill for personnel who are not working on the contract while that contractor employee’s clearance investigation is pending.

10.2 CAC PROCEDURES

Performance under this contract may require the contractor to obtain a CAC.  If CACs are issued under this contract, the contractor shall complete and comply with the following steps to obtain, control, and turn-in Government-issued CACs, as well as establish procedures to always control and account for contractor CACs.  This process is only for the issuance and accountability of CACs.  Contractors may be required to provide additional forms and follow additional procedures for other forms of access and/or background/security checks depending on local site/installation requirements.

    - DLA FORM 1728

a. The contractor is responsible for ensuring that each contractor employee requiring a CAC completes and signs a DLA Form DH 1728.

b. The contractor employee completes Section I, blocks 1-8 and 16 of the DH 1728.  The contractor then forwards the form to the contracting officer (KO) who completes Section I, blocks 9-15 and 17 as the sponsor.

    - DOD FORM 1172-2

    - The contractor must have each contractor employee requiring a CAC complete and sign Sections I and II of the DD Form 1172-2.

(1) For Section II, Block 22 - If the contractor employee signs the DD Form 1172-2 it is then forwarded by the contractor.  This provides verification of the employee information.  If the contractor is submitting a DD Form 1172-2 for him/herself (e.g., the owner), with no higher authority above them for verification, then the signature in Block 22 must be notarized to verify identity.

    - The contractor forwards the DD Form(s) 1172-2 via hand-carry, secure mail, or encrypted email to (NOTE: The contractor may forward these forms individually or as a group):

      - The COR for the contract, if a COR was designated.

      - The KO for the contract, if no COR was designated.

10.2.3 CONTRACTOR RESPONSIBILITIES

a. Each contractor employee must verify his/her account information upon receipt of an email from the Government containing a username and password for the Trusted Associate Sponsorship System (TASS).  The web link for TASS is included in the email.

b. Each contractor employee will receive an email via TASS when his/her account has been approved.  The email will notify the employee to obtain a government issued CAC from the nearest Real-Time Automated Personnel Identification System (RAPIDS) office.  The email will also provide a link that identifies the locations of RAPIDS offices.

c. Each contractor employee shall obtain his/her issued CAC from the RAPIDS office.

d. The contractor, within one (1) business day of issuance, provides written notice via email to the KO for each contractor CAC issued identifying the contractor employee’s name, the date of issuance, and the date of expiration of the CAC.  This notification may be made individually for each contractor employee or together for a group of contractor employees.

e. The contractor establishes procedures to always control and account for Government-issued contractor CACS, to include the following:

    - Ensure contractor CACs issued for this contract are only used for the purpose of performing under this contract.

    - Ensure contractor CACs are secured in a manner that precludes unauthorized use and that recognizes the CAC is the property of the U.S. Government.

    - Ensure contractor employees do not abuse or place holes in their CACs.

    - Ensure contractor employees do not display their CACs in public.

f. If a contractor CAC is lost or stolen:

(1) The contractor employee shall immediately notify the contractor that the contractor CAC has been lost or stolen.

(2) The contractor shall immediately notify the KO, detailing the circumstances regarding the lost or stolen contractor CAC, as follows:

(a) In person, followed within one (1) business day by a written notice via email, or

(b) In writing, via email, or

(c) By telephone, followed within one (1) business day by a written notice via email.

(3) The contractor shall report the lost or stolen CAC card to the local DLA Police/host installation police, who will provide the contractor with a police report.  If there are no local DLA Police/host installation police, or no police report is provided, the contractor shall provide information to the COR/KO, as applicable, detailing the circumstances of how the CAC was lost or stolen.  The COR/KO will provide a memorandum for the contractor employee to support issuance of new CAC.

(4) The contractor has the contractor employee bring the report/memorandum to the nearest RAPIDS office.  If the CAC can be reissued within 24 hours, bring the report/memorandum to the nearest CAC office for reissue.  If the CAC cannot be reissued in 24 hours, follow the steps for a new CAC, beginning at Step 1.

g. If the expiration date for a contractor CAC is prior to completion of the contract and the contractor employee is to continue working under the contract and still requires a CAC, the contractor:

(1) Notifies the COR as follows:

(a) In person, followed within one (1) business day by a written notice via email, or,

(b) In writing, via email, or

(c) By telephone, followed within one (1) business day by a written notice via email.

(2) No less than two (2) weeks prior to the current CAC’s expiration date, completes and complies with the steps above beginning with Step 2 for issuance of a new contractor CAC for the employee.

(3) Brings the expiring CAC to the RAPIDS office for turn-in and receives new CAC.

(4) Notifies the COR once a new CAC has been issued.

h. The contractor/contractor employee turns in any found CACs immediately, via hand carry, to the nearest Federal law enforcement office.

i. The contractor immediately collects all contractor CAC(s) from the contractor employee(s) at:

(1) Contract completion or termination

(2) Termination/Reassignment of an employee (this includes any reason the employee is no longer working for the contractor under the contract or otherwise no longer requires a CAC).

j. The contractor:

(1) For contract completion or termination – within one (1) business day after collecting the contractor CAC(s), arranges for turn-in via one of the below methods:

(a) If the COR/KO is co-located or near enough that in-person transfer of CACs can be arranged, the contractor:

              - Arrange to meet the KO to turn in the collected contractor CAC(s).

              - Hand-carries all collected contractor CACs for turn-in to meet the KO.

              - Completes and signs the Government-Issued Contractor CAC Turn-In Receipt with the KO.  The contractor is provided with a copy of the receipt.

(b) If in-person transfer of CAC cannot be arranged, the contractor:

            - Send, via certified mail, the CACs to the COR/KO.  The contractor includes in the package the Government Issued Contractor CAC Turn-in Receipt, with signed acknowledgement of contractor turn-in.

            - Notifies COR/KO that CACs have been sent via certified mail.

            - Receives a completed copy of the receipt from the COR/KO once the COR/KO has received the CACs.

(2) For termination/reassignment of an employee:

(a) Immediately notify the KO that the employee is no longer working for the contractor under the contract or no longer requires a CAC, as follows:

              - In person, followed within one (1) business day by a written notice via email, or

              - In writing, via email, or

              - By telephone, followed within one (1) business day by a written notice via email.

(b) Follows the turn-in procedures per DL 7036 CAC Turn in form for contract completion or termination, as applicable.

10.3 CONTRACTOR CAC REPORTING REQUIREMENTS

Contractor Reporting Requirements: Monthly the contractor is required to submit a report on all employees working on the contract who have been issued CAC cards and a verification of whether each listed employee still requires a CAC to the KO.  The report must note where changes have occurred (additions or deletions) since the previous month.  Contractor employees who already have a CAC related to another DLA or DoD contract must be included in the monthly report.  Contractor shall note the issuing organization and the CAC expiration date.  Contractor is still responsible for notifying the COR/CO when the individual is no longer working under the subject contract.

10.4 CAC SECURITY COMPLIANCE

The above procedures have been established as a DLA security measure.  Contractors are advised that failure to comply with any of the above requirements will be considered a violation of the terms and conditions of the contract and the Contracting Officer may take action to remedy such violations.  Specifically, failure to safeguard, follow these procedures, including reporting requirements, or turn-in CACs within the established timeframes may result in the following actions, which are in addition to other actions the KO may take under governing law and regulation and the terms and conditions of the contract:

- Immediate work stoppage (issuance of a stop work order), not to be lifted until resolution of CAC issue.

- Disapproval of invoices and delay of payment.

- Withholding of final payment (in accordance with FAR 52.204-9).

- Documentation of CAC Non-Compliance in the Contractor Performance Assessment Reporting System (CPARS).

11.0 PERIOD OF PERFORMANCE

This contract consists of one (1) 30-month Base Period and one (1) 30-month Option Period.  The period of performance of this contract begins on the award/effective date of the contract as reflected in Block 3 of tSF1449 and continues through a 30-month Base Period and any further options or extensions to the contract performance period.